******************************************************

*                                                    *

*   UM9008 Series NDIS Device Driver Installation    *

*                                                    *

******************************************************

Installation of UM9008.SYS onto Windows NT 3.5

===============================================

The UM9008 Series ISA-based Ethernet Adapter can work with Windows NT 3.5,

1994 final release.

Please follow these instructions to load this driver into your Windows NT

system:

1.  Logon as Administrator and select the Network icon from the control

panel.

2.  Select the "Add Adapter..." button and highlight the NE2000 compatible

driver which users want to install. If the Windows NT operating system

does not offer NE2000 driver, choose the "<Other> Requires disk from

manufacturer" item.

3.  Insert the diskette with the appropriate driver in drive A or B.

4.  Follow the instructions appearing on the screen and select the

appropriate drive name (A:\ or B:\) when required to give the

location for the driver and the OEMSETUP.INF file.

For example :

A:\WINNT35 <--- Directory of UMC UM9008 Series Ethernet Driver

5.  After pressing the "Continue" button, confirm the driver diskette with

the message "UMC UM9008 PnP Ethernet Adapter", set the IRQ and/or

I/O Addresses and click on the OK button.

6.  Now, the driver has been installed.  Please select the OK button from

the Network window.  The protocols and driver will bind together.

7.  Reboot