******************************************************************************

*       Genius LAN GE2500III SE PCI Ethernet Card -- NDIS DOS Driver         *

******************************************************************************

A. Diskette Introduction :

==========================

This sub-directory contains the following structures:

MSLANMAN.DOS

DRIVERS

NIF          : Contains the Network Information File (NIF)

that setup needs.

ETHERNET

RTL8029 : Contains the Genius LAN GE25000 III SE PCI

Ethernet Card NDIS DOS driver and

protocol.ini file that could be used

for setup.

B. Installation Guildline:

==========================

1. To change directory to LANMAN.DOS and run Microsoft Lan Manager

SETUP program. In the DOS prompt key in:

C:\LANMAN.DOS>SETUP

2. Press Alt+C to select the "Configuration".

3. Press "D" or move the cursor to "Network Driver" item and press

ENTER.

4. Press Alt+A to select <Add New Config> .

5. In the "Network Adapter Drivers" menu, press Alt+D to select other

driver.

6. Insert the PCI Slave Ethernet Card driver diskette to the floppy disk

A, and press  ENTER.

7. In "Other Network Adapter Drivers" menu, select "Genius LAN GE2500III

SE PCI Ethernet Adapter" and press ENTER.

8. Select the network protocol that you want and press ENTER.

9. To verify the workstation configuration is correct and press ENTER.

10. Select "SAVE" to save the Lan Manager configuration. It will update

the AUTOEXEC.BAT and CONFIG.SYS files.

11. Exit Lan Manager setup program and reboot the system.

--- Sample CONFIG.SYS file ---

:

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTMAN\PROTMAN.DOS /i:C:\LANMAN.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\ETHERNET\RTL8029\PCIND.DOS

:

--- Sample PROTOCOL.INI file ---

[PROTMAN]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

PRIORITY = NETBEUI

[NETBEUI_XIF]

Drivername = netbeui$

SESSIONS = 6

NCBS = 12

BINDINGS = "RTL8029_NIF"

LANABASE = 0

[RTL8029_NIF]

DRIVERNAME = PCIND$

======================================================================