LANtastic 5.0

Introduction:

-------------

LANtastic 5.0 can support two different drivers : NDIS driver and

LANtastic driver.  This document describes the procedure to setup

LANtestic driver for Genius LAN GE2500III SE PCI ethernet card.

Location of Driver:     \ARTISOFT\5.X\PCILAN5.EXE

------------------

Sample Configuration Files:

--------------------------

A. STARTNET (for LANtastic server)

@ECHO OFF

SET LAN_DIR=C:\LANTASTI.NET

SET LAN_CFG=C:\LANTASTI

PATH C:\LANTASTI;%PATH%

SHARE /L:200

REM NEX000 IRQ=3 IOBASE=300 16-BIT VERBOSE ---> Mask this line

PCILAN5 VERBOSE                            ---> Added for RTL8029

AILANBIO

REDIR NAME LOGINS=3

SERVER

NET LOGIN/WAIT \\SERVERNAME NAME

NET LPT TIMEOUT 10

NET POSTBOX

B. STARTNET.BAT  (for LANtastic workstation)

@ECHO OFF

SET LAN_CFG=C:\LANTASTI

PATH C:\LANTASTI;%PATH%

REM NEX000 IRQ=3 IOBASE=300 16-BIT VERBOSE ---> Mask this line

PCILAN5 VERBOSE                            ---> Added for RTL8029

AILANBIO

REDIR WORKSTATION_NAME LOGINS=2

NET LPT TIMEOUT 10

NET POSTBOX

Install GE2500III SE drivers for LANtastic 5.0 : (Using LANtastic driver)

------------------------------------------

1. Before you start with the installation  process, make sure that

the LANtastic operation system (server/workstation) is properly

installed.  Similarly,  your  adapter  should  also be properly

installed in your workstation.

2. Run LANtastic "install" program, and move highlight to "Network

Adapter Install" item,  then ENTER  and  select "Other NETBIOS

Compatible Network Adapter" item.

3. When complete installation,then copy RTL8029 driver"PCILAN5.EXE"

from Genius LAN GE2500III SE driver diskette A:\LANTAS5 to your

LANtastic subdirectory C:\LANTASTI.5 .

4. Modify STARTNET.BAT file as above sample configuration files.

Notes:

------

Installing Multiple LAN Adapters:

If there are multiple network cards on the main board. The user

modify your startnet.bat file as following :

PCILAN5 VERBOSE

AILANBIO

PCILAN5/MPX=D7 VERBOSE

AILANBIO/MPX=D7/ADAPTER=1

The first invocation of PCILAN5.EXE and AILANBIO.EXE loads into

the first adapter using the default MPX number (C7), and adapter

number 0.   The  command line  switches in the second invocation

instruct  the  NETBIOS  to  load into the second card,  (adapter

number 1) using and DOS MPX number D7.  If problems occur,  the

screen will prompt you with error messages. Please refer to your

LANtastic 5.0 manual for details on these messages.