NDIS Dirver Installation Guide for DEC PATHWORKS Client

=============================================================

Introduction

============

This document describes the procedure to install DEC PATHWORKS client

driver for Genius LAN GE2500III SE PCI Ethernet Adapter.

These drivers conform to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedures

=======================

The Genius LAN GE2500III SE PCI Ethernet Adapter is assumed to have

been installed in your computer and physically connect to the

network,  and the DEC "PATHWORKS for DOS" is assumed to be installed

on your hard disk.

1. Insert the DEC PATHWORKS Client Setup Disk #1 into yor floppy

driver (A or B).

2. Under DOS prompt, type "NETSETUP" <Enter>.

3. Unless you have an LK250 keyboard, Select "NO".

4. Select the DECNETT transport option and press <Enter>.

5. Insert the DNET TRNSPRT disk #1 into floppy driver.

6. Specify the destination driver letter (C or D).

7. Are you creating an initial workstation diskette? Select "NO".

8. Type the Node Name and Address (i.e. Node Number), Press <Enter>.

9. Type the Server Name and Address (i.i. Node Number), Press <Enter>.

10. Select "Ethernet" when prompt " Select the network adapter type".

11. Select "(Other NDIS)" when prompt "Select the network adapter".

12. Enter full path and driver name for NDIS driver.

A:\PATHWORK\PCIND.DOS  or  B:\PATHWORK\PCIND.DOS

13. If they are all accurate, select "WRITE KEY DISK" to start insta-

llation. Follow all prompts till completion.

14. At the configuration screen, press F10 to return DOS prompt.

15. Unsing text editor to edit PROTOCOL.INI at DECNET sub-directory and

CONFIG.SYS under root directory as following.

--- Sample CONFIG.SYS file ---

:

DEVICE=C:\DECNET\PROTMAN.SYS /I:C:\DECNET

DEVICE=C:\DECNET\PCIND.DOS

LASTDRIVE=V

:

--- Sample PROTOCOL.INI file ---

:

[IPX4MAC]

DRIVER = IPX$MAC

BINDINGS = RTL8029_NIF

[DATALINK]

DRIVERNAME DLL$MAC

LG_BUFFERS = 16

SM_BUFFERS = 6

OUTSTANDING = 32

HEURISTICS = 0

BINDINGS = RTL8029_NIF

NI_IRQ = 3                     ; Specify IRQ level used by adapter

[RTL8029_NIF]

DRIVERNAME = PCIND$

--- Sample AUTOEXEC.BAT file ---

:

@CALL \DECNET\STARTNET

: