Windows NT 3.1

Introduction:

-------------

This document describes the procedure to install  Windows NT v3.1

driver for Genius LAN GE2500III SE PCI ethernet adapter.

Location of Driver:     \WINNT31\RTL8029.SYS

Installing driver procedure on Microsoft Windows NT :

-----------------------------------------------------

When you are in Windows NT :

1. In the Main group of NT, select the "Control Panel" icon.

2. In the Control Panel window, choose the "Network" icon.

3. In the Network Settings dialog box, choose the "Add Adapter" button.

The Add Network Adapter dialog box appears.

4. In the list of network cards, select "<other> Requires disk from

manufacturer", and then press <Enter> button.

5. Insert the Genius LAN GE2500III SE driver disk in drive A, enter drive

and pathname  A:\WINNT31 which is the path where the driver RTL8029.SYS

and setup file OEMSETUP.INF are located, and then choose the OK button.

6. The screen will appear "Input EthernetID" dialog box which is provide

by RTL8029.SYS driver. This option is only required when you have more

than one Genius LAN GE2500III SE PCI adapters on this computer. Select

SKIP if only one adapter is installed on this computer.

7. NT will then perform the binding process.  If any additional network

software options were installed,  you may be  prompted for  specific

information for these packages.

8. Restarting your system you will acquire network service.

NOTES:

------

1. Installing Multiple LAN Adapters:

Enter Windows NT and follow above setup procedure step 2,in the

"Network Settings" dialog box, choose the "Configure.." button.

The "Input Ethernet ID"  dialog box appears and input adapter's

Ethernet ID. Last step to select OK and close NETWORK SETUP.

Select SKIP if only one adapter is installed on this computer.

2. If you are install Genius LAN GE2500 III SE PCI Ethernet Adapter

in Windows NT at first time.  To use the mini-port driver

(RTL8029.SYS ) on WINDOWS NT 3.1, you must replace the original

system NDIS.SYS file with the A:\WINNT31\NDIS.SYS  file included

in this directory.

For example:

> cd c:\winnt\system32\drivers

> copy a:\winnt31\ndis.sys

or

> cd c:\winnt\system32\drivers

> rename ndis.sys ndis.bak

> copy a:\winnt31\ndis.sys