Microsoft Network Client 3.0 for MS-DOS

Introduction:

-------------

This document describes the procedure to setup the Microsoft Network

Client for DOS driver on Genius LAN GE2500III SE PCI Ethernet adapter.

Location of Driver:   \MSCLIENT\PCIND.DOS

------------------

Sample Configuration Files:

---------------------------

PROTOCOL.INI (install creates this file)

[network.setup]

version=0x3110

netcard=RTL8029,1,RTL8029,1

transport=ms$ndishlp,MS$NDISHLP

transport=ms$netbeui,MS$NETBEUI

lana0=RTL8029,1,ms$netbeui

lana1=RTL8029,1,ms$ndishlp

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NDISHLP

[MS$NDISHLP]

DriverName=ndishlp$

BINDINGS=RTL8029

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=12

BINDINGS=RTL8029

LANABASE=0

[RTL8029]

AdapterID=@52544c190011

DriverName=PCIND$

Installation Procedure :

----------------------

Before you start with the installation procedure, make sure that the

adapter is properly  installed and configured.  Similarly, Make sure

that your  Microsoft Network Client is properly installed.

(If not, The installation utility SETUP.EXE  is located on Microsoft

Network Client v3.0 for MS-DOS Disk. Run SETUP.EXE from Disk and the

SETUP  program screen will appear, and You must complete each of the

questions. These options are specific to your needs. The installation

procedure  will transfer files to a specific directory on the client

and modify existing configuration files to fit your specific needs.)

1. Change to the Microsoft Network Client subdirectory, and run

SETUP.EXE

2. The screen will display some infomation. Press <Enter> .

3. Responds to the screen prompts, selecting "Change Network

Configuration",then press <Enter>.

4. Move the highlight to "Add Adapter", press <Enter>.

5. Select "Network adapter not shown on list below ...".

6. Insert the Genius LAN GE2500III PCI Ethernet Adapter driver

diskette in floppy A,and specify your pathname A:\MSCLIENT ,

then press <Enter>.

7. Follow  screen  instructions  to  complete  the  process. Simply

pressing <Enter> for each screen display will select the default

options. Press <Enter> until you reach the display prompting you

to reboot your system.

NOTES:

------

1. Parameters for the PROTOCOL.INI file:

AdapterID : This keyword is provided  by PCIND, and is required

when  more  than  one  GE2500III SE adapters exist on one

system.

2. Installing Multiple LAN Adapters:

There are two different method to complete it:

(a) Under  DOS  mode, you can modify the AdapterID in file

C:\MSCLIENT\PROTOCOL.INI by yourself.

(b) Or you can also use the SETUP program  and follow  above

setup procedurc step 4,but move the highlight to "Change

Setting,then press <Enter>, the screen will prompt you to

fill AdapterID value .