GE2500III SE Linux Installation guide::

Most Linux does not support network after installation, so recompiling

kernel is usually required to add networking capability.

We suggest the following procedure to install GE2500III SE on linux :

(Hint:RTL8029 is ne2000 compatable.)

step 1: make sure that kernel source code is included

(check /usr/src/linux)

step 2: recompile kernel:

cd to /usr/src/linux, them type 'make menuconfig'

select the following options(mark them as '*')

.enable module support

.networking support

.TCP/IP networking

.network device support

.ethernet

.ne2000/ne1000 support(this will add ne2000 driver).

step 3: .after options are selected, exit and execute

make dep;make clean;make zImage;

.if no serious error happens, the new kernel 'zImage' will be in

/usr/src/linux/arch/i386/boot/zImage.

.copy this file to root.

.modify '/etc/lilo.conf' file to make system boot from this new

kernel image.

.run 'lilo'

.reboot

step 4: when system boots, the driver will be load. check the /proc/pci

file if GE2500III SE device is detected.(use 'dmesg' to check the

boot mseeage)

step 5: run netconfig to setup TCP/IP

(run 'ifconfig' or 'netstat -i' to see if there is a interface

'ne0')