Server Driver for NetWare 4.11

Introduction:

-------------

This document describes the procedure to install the NetWare v4.11

server driver for Genius LAN GE2500III SE ethernet adapter.

Location of Driver:     \NWSERVER\411\PCISRV.LAN

Installaton Procedure :

---------------------

Before you start with the installation  process,  make sure that

the Novell NetWare v4.11 server is properly installed.

Similarly, your adapter should also be properly installed in your

server.

1. Insert the Genius LAN GE2500III Driver Diskette into

drive A and check the contents of subdirectory  \NWSERVER\411,

It should contain the following file :

PCISRV.LAN  Novell NetWare V4.11 Server Driver

PCISRV.LDI  Novell NetWare V4.11 Server Driver Installation

Information File

2. At the  NetWare prompt (indicated by the Server name), run the

INSTALL.NLM program by typing:

server name: LOAD INSTALL <Enter>

3. Select "Maintenance/Selective Install" and press <Enter>.

4. Select  " LAN Driver Options (Configure/Load/...) "  and press

<Enter>.

5. Press the <Ins> key to specify other drivers to install.

6. Press  <F3>  and specify the driver path (A:\NWSERVER\411)

and press <Enter>.

7. The PCISRV.LAN driver should appear in your choice list for the

'Select a  LAN  Driver' field. Choose this  driver to start the

driver loading and  binding  procedure.  This will allow you to

load and bind all 4 frame types supported by NetWare.

8. Add the LOAD and BIND statements you require to the server's

AUTOEXEC.NCF file so that the LAN driver will load automatically

each time the server starts up.

Installation Notes:

-------------------

* Installing Multiple LAN Adapters:

The keyword  "SLOT"  is  provided for multiple LAN adapters in a

single server by the driver PCISRV.LAN.  So, add EtherID in LOAD

commands. For example:

LOAD PCISRV FRAME=Ethernet_802.2 NAME=LAN_A SLOT=1

BIND IPX TO LAN_A NET=11

LOAD PCISRV FRAME=Ethernet_802.2 NAME=LAN_B SLOT=2

BIND IPX TO LAN_B NET=22