LAN Server 2.0/3.0

Introduction:

-------------

This document describes the procedure to install NDIS driver on DOS

LAN Requester and  OS/2 LAN server 2.0 (or 3.0) for Genius GE2500III

SE PCI ethernet adapter.

Location of Driver:  \NDIS\DOS\LAN_S23\PCIND.DOS  (for DOS LAN Requester)

\NDIS\OS2\PCIND.OS2  (for OS/2 LAN Server)

Installing driver procedure on DOS LAN Requester :

--------------------------------------------------

1. Insert the IBM LAN Support Program (LSP) Version 1.30 (or 1.31)

Diskette into your floppy drive A: .

2. Type A:\dxmaid  and press <Enter> .

3. The screen will display the IBM logo. Press <Enter> twice until

the "Environment Information" appears on screen.

4. Responds to the screen prompts, specifying the following values

for each field,use the <Space Bar> to toggle between choices:

Configuration for this computer?          Yes

Use existing configuration information?   Yes

Do you have adapter option diskettes?     Yes

Are you configuring for two adapters?     No

Do you need 802.2 interface support?      Yes

Source for LSP                           A:\

Target for new configuration             C:\LSP

Make  sure  that  the values given above are correct, then press

<Enter> to store your choices.

5. Insert the Genius LAN GE2500III SE PCI Ethernet Card driver diskette

in floppy A,and specify your pathname A:\NDIS\DOS\LAN_S23,then press

ENTER.

6. Follow  screen  instructions  to  complete  the  process. Simply

pressing <Enter> for each screen display will select the default

options. Press <Enter> until you reach the display prompting you

to insert the LSP diskette.

7. Insert the LSP diskette as prompted. Press <Enter> to acknowledge

completion of the action.

8. A screen  showing  the  current  configuration  detected will be

displayed. It should show the  Genius LAN GE2500II SE PCI Ethernet

adapter and the protocols already selected

(i.e., "IBM IEEE 802.2" and "IBM OS/2 NETBIOS").

9.  Press <F4>. to save the new configuration. This completes the LSP

installation.

10. Save the configuration and reboot your computer.

11. Insert the DOS LAN Requester Install Diskette 1 into your floppy

drive.

12. Type A:\INSTALL    <Enter>

13. The IBM logo will again appear on the screen. Press <Enter> to

move on to the next screen.

14. Follow  screen  instructions,  pressing  <Enter>  to move from

screen-to-screen until you are prompted to indicate the pathname

of the DOS LAN Requester directory (C:\DOSLAN ).

15. Specify the pathname (C:\DOSLAN is the default) for the DOS LAN

Requester directory . Then press <Enter> to continue.

16. The program will ask you for the services that you require.

17. Highlight the appropriate option. Select either "Send messages"

or "Send, View, Edit, and Log message" option. (Normally, second

option "Send, View, Edit and Log message" is selected).

18. Specify the PC's ID and domain ID.

19. Follow screen prompts to complete the installation.

Installing driver procedure on OS/2 LAN Server 2.0/3.0 :

------------------------------------------------------

Before you start with the installation  process,  make sure that

OS/2 LAN Server  is properly installed.  Similarly, your adapter

should also be properly installed in your server.

1. Copy PCIND.OS2 and RTL8029.NIF files from your Driver diskette

(\NDIS\OS2) to C:\IBMCOM\MACS.

2. Click the OS/2 LAN Service icon.

3. Select "Installation  and  configuration". The IBM logo should

appear onscreen.

4. Click <OK>.

5. Select <Advance> and press <Enter>.

6. Select "Install or Configuration this Workstation."

7. Select server type, adding a server or domain controller.  The

"install or remove" selections should appear.

8. Select "Configure a component."

10. Click the "Configure" button.

11. Select "Configure workstation."

12. Select "RTL8029 PCI ethernet adapter" when prompted to identify

you Network Adapter. Then click the <Add> button.

13. Select IBM NETBIOS from the list and click the <Add> button.

14. Select <OK> to confirm all selections.

15. Follow screen instructions to complete the process, selecting

<OK> to accept default selections.

16. Continue pressing <OK> until your screen prompts you to backup

old  CONFIG.SYS and  AUTOEXEC.BAT and create a new  CONFIG.SYS

and AUTOEXEC.BAT. Also select <OK> to confirm this.

17. Installation is complete now. Reset your PC by turning it off

and on.

Note:

-----

The DOS version must be bellow DOS 5.0 when install DOS LAN requester

into your system.