LAN Server 4.0

Introduction:

-------------

This document describes the procedure to install NDIS driver on DOS

LAN Requester and OS/2 LAN server 4.0 for Genius LAN GE2500III SE

PCI Ethernet adapter.

Location of Driver:  \NDIS\DOS\LAN_S4\PCIND.DOS  (for DOS LAN Services)

\NDIS\OS2\PCIND.OS2  (for OS/2 LAN Server)

Installing driver procedure on DOS LAN Requester :

--------------------------------------------------

Before you start with the installation  process,  make sure that

DOS LAN Services is properly installed.  Similarly, your adapter

should also be properly installed in your client PC.

1. Copy PCIND.DOS from your Driver diskette (A:\NDIS\DOS\LAN_S4)

to DOS LAN Services subdirectory C:\DLS .

2. Under DOS LAN Requester subdirectory C:\DLS, run INSTALL

3. The  screen  display  some description. If it's correct, move

highlight to "The listed options are correct" item ,then press

ENTER.

4. Move highlight to "Network Card", then press ENTER.

5. Select "Change driver for network card." item, then press ENTER.

6. Select "Network card not shown in list below..", then press ENTER.

7. Insert the Genius LAN GE2500III SE PCI Ethernet Adapter driver

diskette in floppy A,and specify your pathname A:\NDIS\DOS\LAN_S4,

then press ENTER.

8. Genius LAN GE2500III SE PCI Ethernet Card driver is now installed

on your computer, reboot your computer.

NOTES:

------

Parameters for the PROTOCOL.INI file:

AdapterID : This keyword is provided by PCIND, and is required

when  more than one RTL8029  adapters exist on one

system.

Installing driver procedure on OS/2 LAN Server 4.0:

---------------------------------------------------

Before you start with the installation  process,  make sure that

OS/2 LAN Server  is properly installed.  Similarly, your adapter

should also be properly installed in your server.

1. In OS/2 window, Click the "MPTS" icon.

2. The screen will display the IBM Log, press "OK".

3. Select "Install" button.

4. Insert the Genius LAN GE2500III SE PCI Ethernet Card diskette, specify the driver

path A:\NDIS\OS2 and press <Enter>.

5. When complete above step to load  PCIND.OS2  into OS/2 system,

then configure your system.

6. The same step as step 3, select "Configure" button.

7. The screen will display "Configure" dialog box and select "LAN

adapters and protocols" then press "configure" button.

8. The screen will display "LAPS Configuration" dialog box, move

highlight to "Genius LAN GE2500III SE PCI Ethernet Adapter"

item in "Network Adapter" window, and select "NetBios"

protocol. Press "OK".

9. Installation is complete now. Reset your PC by turning it off

and on.