LANtastic 4.0

Introduction:

-------------

This document describes the procedure to setup LANtastic 4.0

driver for Genius LAN GE2500III SE  PCI ethernet card.

Location of Driver:     \ARTISOFT\4.X\PCILAN4.EXE

------------------

Sample Configuration Files:

---------------------------

A. STARTNET (for LANtastic server)

@ECHO OFF

SET LAN_DIR=C:\LANTASTI.NET

SET LAN_CFG=C:\LANTASTI

PATH C:\LANTASTI;%PATH%

SHARE /L:200

REM NEX000 IRQ=3 IOBASE=300 16-BIT VERBOSE ---> Mask this line

PCILAN4 VERBOSE                            ---> Added for GE2500III SE

AILANBIO

REDIR NAME LOGINS=3

SERVER

NET LOGIN/WAIT \\SERVERNAME NAME

NET LPT TIMEOUT 10

NET POSTBOX

B. STARTNET.BAT  (for LANtastic workstation)

@ECHO OFF

SET LAN_CFG=C:\LANTASTI

PATH C:\LANTASTI;%PATH%

REM NEX000  IRQ=3 IOBASE=300 VERBOSE       ---> Mask this line

PCILAN4  VERBOSE                           ---> Added for GE2500III SE

AILANBIO

REDIR WORKSTATION_NAME LOGINS=2

NET LPT TIMEOUT 10

NET POSTBOX

Install Geniusl LAN GE2500III SE Ethernet Card drivers for LANtastic 4.0 :

------------------------------------------

1. Before you start with the installation  process,  make sure that

the LANtastic operation system (server/workstation)  is properly

installed.  Similarly,  your  adapter  should  also  be properly

installed in your workstation.

2. Run LANtastic "install" program , and move highlight to "Network

Adapter Install" item,  then ENTER  and  select "Other NETBIOS

Compatible Network Adapter" item.

3. When have completed installation, then  copy  GE2500III  driver

"PCILAN4.EXE" from Genius LAN GE2500III SE driver diskette

A:\LANTAS4 to your LANtastic subdirectory C:\LANTASTI .

4. Modify STARTNET.BAT file as above sample configuration files.

Notes:

------

Installing Multiple LAN Adapters:

If there are multiple network cards on the main board. The user

modify your startnet.bat file as following :

PCILAN4 VERBOSE

AILANBIO

PCILAN4/MPX=D7 VERBOSE

AILANBIO/MPX=D7/ADAPTER=1

The first invocation of PCILAN4.EXE and AILANBIO.EXE loads into

the first adapter using the default MPX number (C7), and adapter

number 0.   The  command line  switches in the second invocation

instruct  the  NETBIOS  to  load into the second card,  (adapter

number 1) using and DOS MPX number D7.  If problems occur,  the

screen will prompt you with error messages. Please refer to your

LANtastic 4.0 manual for details on these messages.