Version  ETH16.386/v1.0                            Date: June.16, 1994

The programs in this directory can be used to run Windows for

Workgroups 3.11 operating system.

How to Install NDIS Driver

--------------------------

STEP 1:

=======

Proceed to install Windows for Workgroups 3.11. If you have already

installed a Windows for Workgroups 3.11, open the Network Setup Window

to change the network settings.

STEP 2:

=======

When Windows asks for the type of network adapter, it will show a

list of all network adapters supported by Windows. You must select

'Unlisted or Updated Network Adapter'.

STEP 3:

=======

Windows will ask for an OEMSETUP.INF file. Since the OEMSETUP.INF

file is under the driver diskette directory  WFW3.X\3.11, insert

the driver diskette in drive A:

Input the full path name where the OEMSETUP.INF file is located.

(i.e. A:\WFW3.X\3.11\OEMSETUP.INF)

STEP 4:

=======

You will see a Windows dialog menu listing the network adapter

included in the OEMSETUP.INF file. Select the network adapter that

you will be using.

STEP 5:

=======

Windows will copy the NDIS driver file and update the Windows network

system automatically. Be sure to configure the network adapter if you

are using I/O port or IRQ other than the default.

STEP 6:

=======

You can select 'Enhanced Mode NDIS Driver' or 'Real Mode and Enhanced

Mode NDIS Driver' in 'Driver Type' section to use Fiberline NE2000

compatible NDIS 3.0 driver.

STEP 7:

=======

Congratuation!  You are ready for networking.  Please reboot.