. About LAN2M.COM

. Is a MLID (Multiple Link Interface Driver) for Novell DOS ODI driver

. Hardware Specific Module for REPOTEC LAN-2Mx jumperless cards

. Working with Ethernet NE2000 compitable mode

. Auto configuration on board or with configuration file: NET.CFG

. Work with NET.CFG

. Session name: LINK DRIVER LAN2M

. Auto configuration parameters: PORT address, INT interrupt

. NET.CFG Examples: (For multiple same boards concurrently)

==============================

LINK DRIVER LAN2M               (First Link)

PORT    300

INT     3

...

FRAME   ETHERNET_802.3

LINK DRIVER LAN2M               (Second Link)

FRAME   ETHERNET_802.3

LINK DRIVER LAN2M               (Third Link)

PORT    320

INT     5

FRAME   ETHERNET_802.3

==============================

. First Link: Use first board with port 300, IRQ 3,

If port 300 has no board exists, find a board by

scanning I/O ports from low address to high address,

and configuration to port 300, IRQ 3

. Second Link: No parameters assigned, find the second

board and use its on board setting as default.

If no more board exists, use the first board and acts

as a virtual driver.

. Third Link: Find the third board first with port 320,

if found, set its IRQ to 5 if it's not the same, if

the port address has no board on it, scanning and

with counting for the third board to as the target,

if can not find any more boards, return fail messages

for the board cannot be found.

. What is ODI ?

. Open Data-Link Interface

. New improved LAN interface by Novell

. Support multiple network protocols and LAN adapters

. Contains: (list as execution sequence)

. Link Support Layer (LSL.COM)

. MLIDs Multiple Link Interface Drivers (<driver>.COM as this)

. Protocol driver (as IPXODI.COM or ???ODI.COM)

. For workstation to connect/login Novell NetWare Server, you need as

. NetWare Workstation Shell program (as NETx.COM)