# Introduction

The modem software drivers described in this note were built on December 21, 1998.

This is a release candidate version of V.90 drivers for Apollo ISA platform and Mars PCI platform.

Apollo - This version of drivers will run on DSP1642-CC, DSP1641-CC, DSP1643-CH, DSP1644-CH, DSP1641B-CH  ROM code. 56K and V.90 will work only on the CH ROM codes; this is a run time check option.

Mars - this version of drivers will run on DSP1645 and DSP1646 - AB ROM code.

This version of drivers does not require the external SRAMS for K56flex or V.90.

# Changes Since Version 5.35 for Apollo and Mars

98/12/08 16:17:04 trevor

Vendor Specific Change :  Vendor ID 0x1014, 0x1468, 0x1436  :  Win 95, use PME for wake up on ring. Modified the method of suspend and resume for machines that use PME on WIN 95. We now call the functions PM_SetD3 and PM_SetD0, when the modem is suspended or resumed. This fixes the bug where the system had multiple devices sharing the interrupt and would hang when an incoming ring on WIN 95.

98/12/10 09:54:06 zdo

Modem Setup Program :  delmodem.exe : Problem when installing the modem drivers, the installer used to remove all “Unknown Devices”. Added function RemoveUnkownDeviceEx that will look at specific branch of Enum path in registry  ( PCI, ISAPNP, BIOS, ACPI ) for most used Lucent HW Ids or BIOS modem ID’s.  If found as an unknown device they will be removed.  This gets around the problem of a botched install and the modem showing up in unknowns, as well as in most of cases we no longer remove all of the unknown devices.

98/12/10 10:43:19 vr

Win 98/Win 95 : Problem when sending files with line rates less than equal to 2400 bps using Z-modem protocol with Hyperterminal.  The data rate of the dte is slowed down to less than 256 bytes at a time to fix this problem. This change is done only for line rates of <= 2400 bps.  The problem is still there with 300 bps,  but this happens with external modems also with zmodem protocol.

98/12/10 15:01:25 edeng

V.90 – DSP Version 1.58 : Previous release had DSP version 1.62, the change for Cisco servers,  but this change had a problem with certain lines.  Also the change for fast rate change instead of a full-retrain caused some problems on some lines. This version of drivers is the same as version 5.33 of drivers except resetting the lapm counter when  falling back to V.34.

98/12/10 15:14:08 atv

Vendor Specific Change :  Vendor ID 0x1436  : Changed homologation parameters for 0x1436 subsystem id and sub device id 0x0440.

98/12/10 16:06:06 atv

Vendor Specific Change :  Vendor ID 0x1014  : JATE changes. Transmit  for Japan  set to -9 dBm than –15 dBm. Also changed the blacklisting requirements, blacklist after 15 attempts.

98/12/10 16:20:59 vr

Vendor specific change :  For ISA PnP  device ID 0x99 use full-wave ring detect and for ID 0x97 use half-wave ring detect. This change is only made when the country ID is set to Japan.

98/12/11 12:09:44 vr

Win 98/ Win 95 : Bug, low throughput when applications send one byte at a time to the modem driver. The PortWrite function is changed for  applications sending one byte only  at a time. The application is notified immediately when there is space in the DTE transmit queue to take more data.  This change improves the throughput to the actual line

rate if the application writes one byte a time for each transmit notification.

98/12/14 08:52:03 vr

Mars/Apollo V.90 Version 5.36

98/12/16 13:56:01 atv

Homologation : Added Country Enable parameter. This parameters allows the customer to disable individual countries. Default value of CountryEnable set to TRUE, all countries enabled. Please see the homologation document for details. Also messages can be displayed for invalid countries, Pop-up messages if the country is invalid.  The pop-up messages can be enabled thru  and entry in the ltmodem.ini file.

98/12/16 17:28:11 atv

Homologation :  Bug when reporting RING after caller ID for NT5.  The timing is improved after CID detection to report the next RING. This improves the ring reporting timing and fixes the call waiting problem observed with new TAPI in NT5.

98/12/17 13:59:36 edeng

V.90 : Problem on certain lines if the DSP detects and EIA 6 or EIA 7 loop condition. Previously the controller function would not allow the DSP to try V.90 connection on such loop conditions. Now we try V.90 and only if the constellation design fails do we go back to V.34. This change makes V.90 work on some long loops where we might think the loop is a loaded loop.

98/12/17 15:18:22 vr

Win 98/Win 95 : Problem when sending files with line rates less than equal to 2400 bps using Z-modem protocol with Hyperterminal.  A change was made in Version 5.36 to fix this problem, but caused other applications to fail. This changed has been removed.

98/12/18 12:02:32 atv

Homologation : Added Israel and Saudi Arabia country Ids’. Added functions to initialize parameters for Israel and Saudi Arabia. Please see the homologation document for details.

98/12/18 16:13:28 sarkar

Vendor Specific Change :  Vendor ID 0x1014  :  Display  pop up messages when the selected country is unsupported or the modem is not yet approved in that country.

98/12/18 17:17:34 sarkar

Vendor Specific Change :  Vendor ID 0x1014  :  Display pop-up message when an over current function is detected when going off-hook.  The message is contained the “ltmsg.exe” file.

98/12/18 17:39:57 atv

Homologation : CTR 21 – Modified dial tone detection and busy tone detection to cover CTR 21 and national requirements.

98/12/18 18:06:18 trevor

Win 98/ Win 95 : Bug, low throughput when applications send one byte at a time to the modem driver. The PortWrite function is changed for  applications sending one byte only  at a time. The application is notified immediately when there is space in the DTE transmit queue to take more data.  This change improves the throughput to the actual line

rate if the application writes one byte a time for each transmit notification.

This change was not done correctly in Version 5.36. This version fixes this bug and works with all applications.

98/12/21 15:14:08 atv

Vendor Specific Change :  Vendor ID 0x1468  : Changed homologation parameters for 0x1468 subsystem id and sub device id 0x0440. The previous build was overwriting the parameters with CTR 21 parameters. This version fixes this bug.

98/12/21 18:43:50 vr

Mars/Apollo V.90 Version 5.37

# Open Issues/Bugs

Lucent will update the list of all open issues/bugs etc.  Testing is in progress.

**** END ****