Conexant HSF Modem

HSF INF Customization

- Version 2.13.04

Feb, 2000

Prepared by: Conexant
PCD Systems Engineering

Table of Contents

1	Introduction	4

2	Country Parameter Values	4

2.1	T35CODE	4

2.2	NAME	4

2.3	INTCODE	5

2.4	TXLEVEL	5

2.5	RELAYS	5

2.5.1	RELAYS(Standard DAA Only)	5

2.5.2	RELAYS(Smart DAAl Only)	6

2.6	PULSE	8

2.7	RING	8

2.8	TONEPARAMS	9

2.8.1	Primary Tone A	9

2.8.2	Alternate Tone A	10

2.8.3	Voice Call Progress Tone A	10

2.9	SREG	10

2.10	DTMF	10

2.11	THRESHOLD	11

2.12	RLSD	11

2.13	TONE	11

2.14	TIMING	11

2.15	CADENCE	12

2.16	BLACKLISTING	12

2.16.1	List of Blacklisting Parameters to be declared in the INF File	13

2.16.2	Blacklisting Method Descriptions	13

2.16.3	Recommended INF Blacklisting values	14

2.17	CALLERID	20

2.18	SPEEDADJUST	21

2.19	FLAGS	21

3	OEM Parameter Values	22

3.1	ATI Subkey Values	22

3.1.1	ATI0	22

3.1.2	ATI1	22

3.1.3	ATI3	22

3.1.4	ATI4	22

3.1.5	ATI6	22

3.1.6	ATI7	23

3.2	ATPLUSID Subkey Values	23

3.2.1	AT+GCAP	23

3.2.2	AT+GMI	23

3.2.3	AT+GMM	23

3.2.4	AT+GMR	23

3.2.5	AT+GOI	23

3.2.6	AT+GSN	23

3.2.7	AT+FMI	23

3.2.8	AT+FMM	23

3.2.9	AT+FMR	23

3.3	SREGS	23

3.4	AT+MSSTR	24

3.5	AT+MS	24

3.6	V25TER	24

3.6.1	+IPR	25

3.6.2	+ILRR	25

3.6.3	+MR	25

3.6.4	+ESR	25

3.6.5	+EFCS	25

3.6.6	+ER	25

3.6.7	+DR	25

3.6.8	A8E Related Parameters	25

3.6.9	+A8E Parameters	25

3.6.10	+DS Parameters (Data Compression)	25

3.6.11	+EB Parameters (Break Handling in Error Control Operation)	26

3.6.12	+ES Parameters (Error control and Synchronous Mode Selection)	26

3.6.13	+ESA Parameters (Synchronous Access Mode)	26

3.6.14	+ETBM Parameters (Call Termination Buffer Management)	26

3.6.15	+FIT (DTE Inactivity Timeout)	26

3.6.16	+ICF  (Char Format Options)	26

3.6.17	+IFC (DTE Modem Local Flow Control)	27

3.6.18	+ITF (Transmit Flow Control Thresholds)	27

3.6.19	MISC AT Commands	27

3.7	CONSTTONEPARAMS	27

3.8	AT_MISC_DEF	28

3.9	MISC_VOICE_PARMS	28

3.10	CADENCE	29

3.11	AUDIO_AEC	29

3.12	MOD_THRESHOLD	30

3.13	CALLWAITING_PARMS (TWO-LINE only)	30

3.14	AT_MISC_CELL_DEF (WIRELESS only)	31

3.15	FLAGS	31

3.16	PROPERTIES	32

3.17	DaaType	32

3.18	SmartDAAParams (Smart DAA Only)	32

3.18.1	SmartDAA Flags	33

3.19	SPKR_MUTE_DELAY	34

3.20	DTMF_COMP_LEVEL	34

3.21	HwData	35

4	Top Level Modem Parameters Values	35

4.1	Buffers	36

4.2	BuffersDelay	36

4.3	BondOption	36

# Introduction

This document refers to HSF (software) modem. Within the HSF modem are various family members; they are TWO-LINE, RIPTIDE, and WIRELESS.

The information applies to all modems, unless explicitly stated otherwise.

The HSF modem is a plug and play modem, which writes a set of parameters to the Windows registry upon installation.  The registry information is copied from an INF file that exists on the HSF installation disk.  The HSF INF file contains several sections, which get transferred to the registry.  These include:

- Country

- OEM

- Modem_HW

This document describes in detail all the elements that constitute the “Country” and “OEM” sections of the INF file.  This will enable an HSF customer to modify modem parameters per country in order to satisfy country certification requirements and to adjust for OEM-specific hardware characteristics.  Also, by replicating the “Country” structure, customers can add into the modem support for any number of countries provided the 64K limit imposed on INF file size is not exceeded.

Unless otherwise noted, all the values are entered in hexadecimal.  The Microsoft registry format requires that the Most Significant Byte (in a 2-byte word) be stored first.  For example, the T.35 value for USA is stored as “B5 00”.

# Country Parameter Values

- Location of Key in WINDOWS 9X Registry is

- HKEY_LOCAL_MACHINE \ System \ CurrentControlSet \ Services \ Class \ Modem \ [XXXX] \ Country \ [YYYY]

[XXXX]		-	PnP modem key number

[YYYY]		- 	Country key number

In the Country key is a Registry value called “CURRENT” which is a T35 Code that is used to declare which Country is to be used by the MODEM.

## T35CODE

- Parameter indicates the County Code of the country whose parameters are being specified.

- Note: When setting this value to 0xFFFF in the INF, during installation, the Country Selection Application (CSA) will automatically define the modem’s T.35 Country Code to the Country of Use defined by the OS.

- Defined Values

2 Bytes				Defines T.35 Country Code (0-FE)

## NAME

- Parameter designates the ASCII name of the country specified by the T35CODE parameter.

- Defined Values

CHAR[]		-		Defines Country Name (maximum length 30)

## INTCODE

- Parameter reflects the access code required in order to place an international call from the country of origin.

- Defined Values

CHAR[]		-		International access dial code (maximum length 10)

## TXLEVEL

- Transmit Level Parameters

Specified values reflect negative numbers,  i.e. a declaration of (0f)16  in the INF file implies a value of -15 dBm and a declaration of (01)16 implies a value of -1 dBm.

- HSF accounts for a 3V configuration and a 5V configuration.

- In the 3V configuration, DTMF transmit levels are in the range of  -4 dBm to -11 dBm.

- In the 5V configuration, DTMF transmit levels are in the range of 0 dBm to -7 dBm.

DTMF levels are in declared in 0.1 units i.e. a declaration of (28)16  in the INF file implies a value of –4.0 dBm and a declaration of (14)16 implies a value of –2.0 dBm.

- Defined Values

1 Byte		-		Specifies Minimum data transmit level (-dBm)

1 Byte		-		Specifies Maximum data transmit level (-dBm)

1 Byte		-		Specifies Default data transmit level (-dBm)

1 Byte		-		Specifies Minimum fax transmit level (-dBm)

1 Byte		-		Specifies Maximum fax transmit level (-dBm)

1 Byte		-		Specifies Default fax transmit level (-dBm)

1 Byte		-		Specifies Minimum voice transmit level (-dBm)

1 Byte		-		Specifies Maximum voice transmit level (-dBm)

1 Byte		-		Specifies Default voice transmit level (-dBm)

1 Byte		-		Specifies Transmit level offset +/- -dBm and is used to adjust 								for OEM hardware variations.

1 Byte		-		Specifies transmit level (-dBm) of low DTMF frequency.

1 Byte		-		Specifies transmit level (-dBm) of high DTMF frequency.

## RELAYS

### RELAYS(Standard DAA Only)

- Relay Control Parameters used to map the ASIC. There are 16 GPIO’s from the ASIC which are to be defined by the

OEM. GPIO’s are active LOW.

- Defined Values

2 Bytes	 -	Defines a mask of 16 bits to determine which of the 16 GPIO’s on the ASIC are allowed to be toggled by the HCF driver code.  The remaining GPIO’s are off limits to the HSF code and thus can be used by the HSF OEM customer for any desired application.  For example, a value of (07)16  = (0000 0111)2 is a mask which will allow only GPIO2, GPIO1,  and GPIO0 to be toggled by the HSF drivers..

2 Bytes	-	Defines the default value of the GPIO register set up upon power up.

2 Bytes	-	Defines the value of the GPIO register required to set the modem into the  ModemOffHook_PhoneToLine state.

2 Bytes	-	Defines the value of the GPIO register required to set the modem into the ModemOffHook_PhoneOffLine state.

2 Bytes	-	Defines the value of the GPIO register required to set the modem into the ModemOnHook_PhoneToLine_wCID state.

2 Bytes	-	Defines the value of the GPIO register required to set the modem into the ModemOnHook_PhoneToLine_woCID state.

2 Bytes	-	Defines the value of the GPIO register required to set the modem into the ModemOnHook_PhoneOffLine_wCID state.

2 Bytes-	Defines the value of the GPIO register required to set the modem into the ModemOnHook_PhoneOffLine_woCID state.

2 Bytes-	Defines the value of the GPIO register required to set the modem into the

ModemOffHook_PulseMake state.

2 Bytes	-			Defines the value of the GPIO register required to set the modem into the

ModemOffHook_PulseBreak state.

2 Bytes		-		Defines the value of the GPIO register required to set the modem into the

ModemOffHook_PulseSetup state.

2 Bytes		-		Defines the value of the GPIO register required to set the modem into the

ModemOffHook_PulseClear state.

### RELAYS(Smart DAAl Only)

Relay Control Parameters used to map the DAA configuration control register.

- Defined Values

2 Bytes		Defines a mask of 16 bits to determine the 16 valid bits' field.

2 Bytes		Defines the default value of the configuration register set up upon power up.

2 Bytes		Defines the value of the configuration register required to set the modem into the ModemOffHook_PhoneToLine state.

2 Bytes		Defines the value of the configuration register required to set the modem into the ModemOffHook_PhoneOffLine state.

2 Bytes		Defines the value of the configuration register required to set the modem into the ModemOnHook_PhoneToLine_wCID state.

2 Bytes		Defines the value of the configuration register required to set the modem into the ModemOnHook_PhoneToLine_woCID state.

2 Bytes		Defines the value of the configuration register required to set the modem into the ModemOnHook_PhoneOffLine_wCID state.

2 Bytes	Defines the value of the configuration register required to set the modem into the ModemOnHook_PhoneOffLine_woCID state.

2 Bytes	Defines the value of the configuration register required for setting the modem into the ModemOffHook_PulseMake state.

2 Bytes	Defines the value of the configuration register required for setting the modem into the ModemOffHook_PulseBreak state.

2 Bytes	Defines the value of the configuration register required for setting the modem into the ModemOffHook_PulseSetup state.

2 Bytes	Defines the value of the configuration register required for setting the modem into the ModemOffHook_PulseClear state.

Bit Map of the Smart DAA control register

| 15 | CON |
| --- | --- |
| 14 | IMP1 |
| 13 | IMP0 |
| 12 | DCLoop2 |
| 11 | DCLoop1 |
| 10 | DCLoop0 |
| 9 | RMake1 |
| 8 | RMake0 |
| 7 | LSD GPIO2 |
| 6 | MOV |
| 5 | LSD GPIO0 |
| 4 | HS |
| 3 | RD/CID |
| 2 | EICO |
| 1 | TXM |
| 0 | OH |

Explanation of the Smart DAA control register

| OH | On / Off hook control bit |
| --- | --- |
| TXM | Match to TXM in MCR register |
| EIC0 | Match to EIC0 in DCDAC register |
| RD/CID | Match to RD/CID use in MonADC register |
| LSD GPIO0 | Match to GPIO0 bit , which is bit 4 in GPIOD register in LSD |
| HS | Handset exclude bit control |
| MOV | To be used to control voltage spikes when pulse dialing. (Set only when pulse dialing) |
| LSD GPIO2 | Match to GPIO2 bit , which is bit 6 in GPIOD register in LSD |
| RMAKE | Make resistor select 0: 100 ohms make resistor 1: 200 ohms make resistor 2: 300 ohms make resistor 3: 400 ohms make resistor The driver should use this value as reference value to adjust the DAC to present this reference value to the line. |
| DC-Loop[2:0] | DC loop current select 0: Select the DC loop curve 0 as defined in the SmartDAAParams of the OEM section 1: Select the DC loop curve 1 as defined in the SmartDAAParams of the OEM section 2: Select the DC loop curve 2 as defined in the SmartDAAParams of the OEM section 3: Select the DC loop curve 3 as defined in the SmartDAAParams of the OEM section 4: Select the DC loop curve 4 as defined in the SmartDAAParams of the OEM section |
| IMP[1:0] | Impedance select based on the values defined in the SmartDAAParams of the OEM section 0: set to use the Impedance 0 value defined in the SmartDAAParams of the OEM section 1: set to use the Impedance 1 value defined in the SmartDAAParams of the OEM section 2: set to use the Impedance 2 value defined in the SmartDAAParams of the OEM section 3: set to use the Impedance 3 value defined in the SmartDAAParams of the OEM section |
| CON | Continuous curve fitting control |

## PULSE

- Pulse Dial Parameters

Definition of terms:

Make Time	-	The portion of the pulse dialing pulse (in ms) in which the dialing relay is in the “closed circuit” position.

Break Time	-	The portion of the pulse dialing pulse (in ms) in which the dialing relay is in the “open circuit” position.

- Defined Values

1 Byte				Defines Pulse Dial Mode. Value maps the number of Pulses to be generated

for a given dialing digit.

Number of Pulses for a dialing digit at various Pulse Dial Modes

|  | Number of Pulses for Dial Mode |  |  |
| --- | --- | --- | --- |
| Dialing Digit | 0 | 1 | 2 |
| 0 | 10 | 10 | 1 |
| 1 | 1 | 9 | 2 |
| 2 | 2 | 8 | 3 |
| 3 | 3 | 7 | 4 |
| 4 | 4 | 6 | 5 |
| 5 | 5 | 5 | 6 |
| 6 | 6 | 4 | 7 |
| 7 | 7 | 3 | 8 |
| 8 | 8 | 2 | 9 |
| 9 | 9 | 1 | 10 |

1 Byte		-		Defines the mapping of the &P0 command. Default value is 0.

1 Byte		-		Defines the mapping of the &P1 command. Default value is 1.

1 Byte		-		Defines the mapping of the &P2 command. Default value is 2.

1 Byte		-		Defines the mapping of the &P3 command. Default value is 3.

1 Byte		-		Defines Pulse Make Offset. Value indicates a +/- time adjustment in

milliseconds of the current Make Time Value.  Note that an adjustment is

independent of any change in the Pulse Break Time.

1 Byte		-		Defines Pulse Break Offset. Value indicates a +/- time adjustment in

milliseconds of the current Break Time Value. Note that the adjustment is

independent of any change in the Pulse Make Time.

1 Byte		-		Defines Pulse Dial Setup Time to be used by some countries as a

length of time in milliseconds to set DAA to GPIO_ModemOffHook_PulseSetup

1 Byte		-		Defines Pulse Dial Clear Time to be used by some countries as a

length of time in milliseconds to set DAA to GPIO_ModemOffHook_PulseClear

2 Bytes		-		Defines InterDigit Time. Value reflects the delay time in milliseconds between

consecutive digit transmissions.

2 Bytes		-		Defines Line mute delay time. Value reflects the delay time in milliseconds to set

the line mute relay (OFFHOOK_PULSESETUP) during off hook switch.

## RING

- Ring Detection Parameters

- Defined Values

1 Byte	Defines maximum allowable ring frequency that will be detected (Hz)

1 Byte	Defines minimum allowable ring frequency that will be detected (Hz)

2 Byte	Defines length of time for ring signal to be considered valid. Value reflects the length of time in milliseconds in which the ring that is being received by the modem is required to be stable in order for it to be detected as the beginning of a ring.

2 Byte	Defines minimum time before answering.  Defines the minimum time in milliseconds that the modem must wait after detecting a valid beginning of a ring signal before it goes off hook to begin answering.

2 Byte	Defines minimum delay between rings. Value reflects the minimum delay time in milliseconds between consecutive Rings in order for the two Rings to be considered  separate Rings.

2 Byte	Defines maximum delay between rings. Value reflects the maximum delay time between consecutive Rings in which the later Ring is to be considered part of the same Call as the earlier Ring.

## TONEPARAMS

Tone Detect Parameters used by the HSF Data Pump.

Defined Values

### Primary Tone A

2 Bytes	Defines the Center Frequency of the primary tone detector. Value represents the frequency in Hz.

2 Bytes	Defines the Bandwidth of the primary tone detector. Value represents a range in Hz (with the Center Frequency in the middle. Tones within the specified range should be detected (e.g. if Center Frequency is 450 and the Bandwidth is 400, tones in the range of 250-650Hz are detected).

4 Bytes	Defines primary filter Response Time. Value reflects the minimum time in milliseconds that takes the detector to detect a tone. Longer response time provides more accurate tone detection. Normally, the default value for this parameter should not be changed.

2 Bytes	Defines primary THRESHU. Value represents an upper limit level in dBm. If the

tone level that is being detected  exceeds the value of THRESHU, then the tone detect flag is SET. This value will be overwritten by the values in the THRESHOLD country key. The value should be greater than the Response Time.

2 Bytes	Defines primary THRESHL. Value represents a lower limit level in dBm. If the tone

level falls below THRESHL then the tone detect flag is RESET.  This value will be overwritten by a value derived from the values in the THRESHOLD country key. This value should be greater than the Response Time.

4 Bytes	Reserved.

4 Bytes	Defines primary tone ON stability time. Value reflects the minimum time in milliseconds for which the modem must detect continuous tone in order for the “Tone On” event to be reported.

4 Bytes	Defines primary tone OFF stability time. Value reflects the minimum time in milliseconds for which the modem must detect continuous tone non-existence in order for the “Tone Off” event to be reported.

### Alternate Tone A

2 Bytes				Defines the Center Frequency of the alternate tone detector.

2 Bytes				Defines the Bandwidth of the alternate tone detector.

4 Bytes				Defines the Response Time of the alternate tone detector.

2 Bytes				Defines the THRESHU of the alternate tone detector.

2 Bytes				Defines the THRESHL of the alternate tone detector.

4 Bytes				Reserved.

4 Bytes				Defines alternate tone ON stability time.

4 Bytes				Defines alternate tone OFF stability time.

### Voice Call Progress Tone A

2 Bytes				Defines the Center Frequency of the voice call-progress tone detector.

2 Bytes				Defines the Bandwidth of the voice call-progress tone detector.

4 Bytes				Defines the Response Time of the voice call progress tone detector.

2 Bytes				Defines the THRESHU of the voice call-progress tone detector.

2 Bytes				Defines the THRESHL of the voice call-progress tone detector.

4 Bytes				Reserved.

4 Bytes				Defines voice call-progress tone ON stability time.

4 Bytes				Defines voice call-progress tone OFF stability time.

## SREG

- BYTE [10][4]

- S Registers Limiting Parameters

Used to set the limits and factory default value of an S Register.

Allows for 10 S Registers to be declared.

Each parameter is 4 Bytes and declared in the following format:

Byte 0 - S Register number.

Byte 1 - Limiting minimum value.

Byte 2 - Limiting maximum value.

Byte 3 - Default value.

## DTMF

- DTMF Dial Parameters

- Defined Values

2 Bytes	Defines the required ON time for a dialed DTMF. Value reflects the required time in milliseconds for the DTMF signal to remain ‘high’ during the dialing of a DTMF digit. If value is 0 then the DTMF ON time is the value in S11.

2 Bytes	Defines the required length of time between DTMF digits. Value reflects the required delay time in milliseconds between two consecutive digit transmissions. If value is 0 then the interdigit delay is the value in S11.

## THRESHOLD

Threshold Parameters: Used only when DaaType is Smart DAA (see 3.17).

- Defined Values

14 Bytes			Reserved

2 Bytes				Defines Ring threshold in percent of duty cycle. (Ex: 70% -> 700d = 02BCh)

2 Bytes				Reserved

2 Bytes				Defines CID threshold.

## RLSD

- RLSD Threshold Adjustment Parameter

- Defined Values

4 Bytes				Defines a +/- dB offset from the selected RLSD threshold.

## TONE

- Tone Detection Parameters

- Defined Values

2 Bytes	Defines dial tone stability time. Value reflects the minimum time in milliseconds for which the modem must detect continuous dial tone in order for dial tone to be considered valid. This value is used for cadencing of the dial tone in addition to the dial tone cadence parameters, which are defined in the CADENCE section.

2 Bytes	Defines W dial tone stability time. Value reflects the minimum time in milliseconds for which the modem must detect a continuous dial tone after encountering a W parameter in the dial string for the tone to be considered a valid secondary dial tone. This value is used for cadencing of the dial tone in addition to the dial tone cadence parameters, which are defined in the CADENCE section.

2 Bytes	Reserved.

2 Bytes 	Reserved

1 Byte	Defines the dial tone wait time before non-blind dialing. In the case of blind dialing the value in S6 register is used.

## TIMING

- Timing Parameters for redialing and billing

- Defined Value

2 Bytes	Defines minimum delay when attempting an outgoing call to the same number that was previously attempted. Value reflects the delay time in milliseconds required by the modem to wait after hang-up before initiating another call.

2 Bytes	Defines minimum delay when attempting an outgoing call to a different number than that was previously attempted. Value reflects the delay time in milliseconds required by the modem to wait after hang-up before initiating another call.

2 Bytes	Defines minimum delay when attempting an outgoing call after an incoming call. Value reflects the delay time in milliseconds required by the modem to wait after hang-up before initiating another call.

2 Bytes	Defines billing delay. Value reflects a minimum delay between the time the modem goes off hook and the time when the modem begins answering the call.

## CADENCE

- Tone Cadence Parameters

- Defined Values

1 Byte				Defines number of phases (1,2). Value reflects number of different Phases (pulse)

that identifies a tone’s cycle.

36 Bytes*			Defines cadence for Dialtone

36 Bytes*			Defines cadence for Busy

36 Bytes*			Defines cadence for Ringback

36 Bytes*			Defines cadence for Congestion

*Parameter declared in the following format:

Minimum ON time (ms);  Phase 1			-	4 Bytes

Maximum ON time (ms); Phase 1			-	4 Bytes

Minimum OFF time (ms); Phase 1			-	4 Bytes

Maximum OFF time (ms); Phase 1			-	4 Bytes

Minimum ON time (ms);  Phase 2 (if used)		-	4 Bytes

Maximum ON time (ms); Phase 2 (if used)		-	4 Bytes

Minimum OFF time (ms); Phase 2 (if used)		-	4 Bytes

Maximum OFF time (ms); Phase 2 (if used)		-	4 Bytes

Number of cycles needed for detection (if used)		-	4 Bytes

## BLACKLISTING

Three control flags are used to customize the blacklisting implementation as needed

- Enable permanent blacklisting -

Enabling of this flag causes the HSF to use permanent blacklisting for all method. If flag is disabled the delayed blacklisting is used.

- Reset on success -

Enabling of this flag will allow the resetting of a number’s blacklisting record upon a successful call attempt with that number.

- Distinct failures -

Enabling of this flag will allow for the failure conditions to be treated distinctly. When flag is set a failure type will contain a blacklisting record that is independent of all other failure types. Flag is used in cases when the failure types are assigned different

blacklisting methods.

Manual Intervention implies a power up or reset of the PC has just occurred, therefore - prepare to clear all blacklisting records.

### List of Blacklisting Parameters to be declared in the INF File

Defined Values

1 Byte		Defines method number to be used for occurrence of No Dialtone

1 Byte		Defines method number to be used for occurrence of Key Abort

1 Byte		Defines method number to be used for occurrence of Busy

1 Byte		Defines method number to be used for occurrence of No Connection

1 Byte		Defines method number to be used for occurrence of No Answer

1 Byte		Defines method number to be used for occurrence of Voice Answer

2 Bytes		Defines Delay1 to be used for Inter Call Delay upon failure when redialing the

same number.

2 Bytes		Defines Delay2 to be used for Inter Call Delay upon failure when redialing a

different number.

2 Bytes		Defines Delay3 to be used by Method 2

2 Bytes		Defines Delay4 to be used by Method 3

2 Bytes		Defines Count1 to be used by Method 2

2 Bytes		Defines Count2 to be used by Method 3

1 Byte		Blacklisting Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

bit7	-	reserved

### Blacklisting Method Descriptions

#### Method 1

Countries:

Germany, Portugal, Norway

Parameters:

Delay1 -  	Defines the required Inter Call Delay time upon failure

when attempting to redial the same number.

Delay2 -  	Defines the required Inter Call Delay time upon failure when

attempting to redial a different number.

Description:

Method consists of only Inter Call Delays upon failure. Blacklisting Inter Call Delay time is an extension of the Inter Call Delay time indicated in the Billing section of the Country Parameters. If the Inter Call Delay time in the Billing section is 3 sec and the Blacklisting Inter Call Delay is 5 sec, the driver will Delay the larger of the two delay times thus the Inter Call Delay time as seen by the user will be 5 sec.

#### Method 2

Countries:

Japan, Spain		-	Delayed Blacklisting

Belgium, France	-	Permanent Blacklisting

Parameters:

Delay1	- Defines the required Inter Call Delay time upon failure

when attempting to redial the same number.

Delay2 	- Defines the required Inter Call Delay time upon failure

when attempting to redial a different number.

Delay3	- Defines a delay time.

Count1	- Defines maximum number of allowed consecutively failed

call attempts.

Description:

Method consists of a primary condition and an Inter Call Delay. Inter Call Delay is only implemented  when the primary condition is false.

The primary condition is true when  Count1 number of failed attempts is exceeded in any sliding period of Delay1 time.  Method allows for a permanent or delayed blacklisting implementation which is indicated though a control flag. In the permanent blacklisting implementation, the user is prevented from dialing the blacklisted number until manual intervention by the user. In the delayed blacklisting implementation, the number is delayed for a period of time of (Delay3 - time of Count1 attempt past). Note that the delayed implementation does not hold the command line, thus user is able to dial a different number.

Method also incorporates an Inter Call Delay feature declared by Delay1 and Delay2.

#### Method 3

Countries:

Portugal, Italy, Switzerland, Austria

Parameters:

Delay1	- Defines the required Inter Call Delay time upon failure

when attempting to redial the same number.

Delay2 	- Defines the required Inter Call Delay time upon failure

when attempting to redial a different number.

Delay4	- Defines a Delay time

Count2	- Defines maximum number of allowed consecutively failed

call attempts.

Description:

Method consists of a primary condition and an Inter Call Delay. Inter Call Delay is only implemented  when the primary condition is false.

The primary condition is true when Count5 number of failed attempts is exceeded. Method allows for a permanent or delayed blacklisting implementation which is indicated though a control flag. In the permanent blacklisting implementation, the user is prevented from dialing the blacklisted number until manual intervention by the user. In the delayed blacklisting implementation, the number is delayed for a period of time of Delay7. Note that the delayed implementation does not hold the command line, thus the  user is able to dial a different number.

Method also incorporates an Inter Call Delay feature declared by Delay1 and Delay2.

### Recommended INF Blacklisting values

#### Japan

3 failed call attempts allowed in any sliding window of 3 minutes. If condition is exceeded, number is delayed by a time of (3 minus the time elapsed since start of 1ST call in sliding window) minutes

HKR, Country\XXXX, BLACKLISTING, 1, \

02,\		;BlackListing Method for NoDialTone

02,\		;BlackListing Method for KeyAbort

02,\		;BlackListing Method for Busy

02,\		;BlackListing Method for NoConnection

02,\		;BlackListing Method for NoAnswer

02,\		;BlackListing Method for VoiceAnswer

05,00,\		;BlackListingDelay1

05,00,\		;BlackListingDelay2

B4,00,\		;BlackListingDelay3

00,00,\		;BlackListingDelay4

03,00,\		;BlackListingCount1

00,00,\		;BlackListingCount2

02		;BlackListing Control Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

#### Austria

Delay of 30 sec between failed attempts when dialing the same number.

After 12 consecutive failed attempt, number is permanently blacklisted until manual intervention.

HKR, Country\XXXX, BLACKLISTING, 1, \

03,\		;BlackListing Method for NoDialTone

03,\		;BlackListing Method for KeyAbort

03,\		;BlackListing Method for Busy

03,\		;BlackListing Method for NoConnection

03,\		;BlackListing Method for NoAnswer

03,\		;BlackListing Method for VoiceAnswer

1E,00,\		;BlackListingDelay1

05,00,\		;BlackListingDelay2

00,00,\		;BlackListingDelay3

00,00,\		;BlackListingDelay4

00,00,\		;BlackListingCount1

0C,00,\		;BlackListingCount2

03		;BlackListing Control Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

#### Belgium

Delay of 60 sec between failed attempts when dialing same number.

4 consecutive failed attempts are allowed in any sliding window of 1 hour. If the condition is exceeded, then the number is permanently blacklisted until manual intervention.

HKR, Country\XXXX, BLACKLISTING, 1, \

02,\		;BlackListing Method for NoDialTone

02,\		;BlackListing Method for KeyAbort

02,\		;BlackListing Method for Busy

02,\		;BlackListing Method for NoConnection

02,\		;BlackListing Method for NoAnswer

02,\		;BlackListing Method for VoiceAnswer

3C,00,\		;BlackListingDelay1

05,00,\		;BlackListingDelay2

10,0E,\		;BlackListingDelay3

00,00,\		;BlackListingDelay4

04,00,\		;BlackListingCount1

00,00,\		;BlackListingCount2

03		;BlackListing Control Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

#### Germany

Delay of 30 sec between failed attempts when dialing the same number.

HKR, Country\XXXX, BLACKLISTING, 1, \

01,\		;BlackListing Method for NoDialTone

01,\		;BlackListing Method for KeyAbort

01,\		;BlackListing Method for Busy

01,\		;BlackListing Method for NoConnection

01,\		;BlackListing Method for NoAnswer

01,\		;BlackListing Method for VoiceAnswer

1E,00,\		;BlackListingDelay1

05,00,\		;BlackListingDelay2

00,00,\		;BlackListingDelay3

00,00,\		;BlackListingDelay4

00,00,\		;BlackListingCount1

00,00,\		;BlackListingCount2

02		;BlackListing Control Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

#### France

Delay of 60 sec between failed attempts when dialing same number.

6 consecutive failed attempts are allowed in any sliding window of 1 hour. If the condition is exceeded, then the number is permanently blacklisted until manual intervention.

HKR, Country\XXXX, BLACKLISTING, 1, \

02,\		;BlackListing Method for NoDialTone

02,\		;BlackListing Method for KeyAbort

02,\		;BlackListing Method for Busy

02,\		;BlackListing Method for NoConnection

02,\		;BlackListing Method for NoAnswer

02,\		;BlackListing Method for VoiceAnswer

3C,00,\		;BlackListingDelay1

05,00,\		;BlackListingDelay2

10,0E,\		;BlackListingDelay3

00,00,\		;BlackListingDelay4

06,00,\		;BlackListingCount1

00,00,\		;BlackListingCount2

03		;BlackListing Control Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

#### Portugal

Delay of 60 sec between failed attempts when dialing the same number.

After 5 consecutive failed attempt, number is delayed by 1 hour.

HKR, Country\XXXX, BLACKLISTING, 1, \

03,\		;BlackListing Method for NoDialTone

03,\		;BlackListing Method for KeyAbort

03,\		;BlackListing Method for Busy

03,\		;BlackListing Method for NoConnection

03,\		;BlackListing Method for NoAnswer

03,\		;BlackListing Method for VoiceAnswer

3C,00,\		;BlackListingDelay1

05,00,\		;BlackListingDelay2

00,00,\		;BlackListingDelay3

10,0E,\		;BlackListingDelay4

00,00,\		;BlackListingCount1

05,00,\		;BlackListingCount2

03		;BlackListing Control Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

#### Spain

4 failed call attempts allowed in any sliding window of 1 hour. If condition is exceeded, number is delayed by a time of (60 minus the time elapsed since start of 1ST call in sliding window) minutes

HKR, Country\XXXX, BLACKLISTING, 1, \

02,\		;BlackListing Method for NoDialTone

02,\		;BlackListing Method for KeyAbort

02,\		;BlackListing Method for Busy

02,\		;BlackListing Method for NoConnection

02,\		;BlackListing Method for NoAnswer

02,\		;BlackListing Method for VoiceAnswer

05,00,\		;BlackListingDelay1

05,00,\		;BlackListingDelay2

10,0E,\		;BlackListingDelay3

00,00,\		;BlackListingDelay4

04,00,\		;BlackListingCount1

00,00,\		;BlackListingCount2

02		;BlackListing Control Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

#### Italy

Delay of 60 sec between failed attempts when dialing the same number.

After 4 consecutive failed attempt, number is permanently blacklisted until manual intervention.

HKR, Country\XXXX, BLACKLISTING, 1, \

03,\		;BlackListing Method for NoDialTone

03,\		;BlackListing Method for KeyAbort

03,\		;BlackListing Method for Busy

03,\		;BlackListing Method for NoConnection

03,\		;BlackListing Method for NoAnswer

03,\		;BlackListing Method for VoiceAnswer

3C,00,\		;BlackListingDelay1

05,00,\		;BlackListingDelay2

00,00,\		;BlackListingDelay3

00,00,\		;BlackListingDelay4

00,00,\		;BlackListingCount1

04,00,\		;BlackListingCount2

03		;BlackListing Control Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

#### Norway

Delay of 60 sec between failed attempts when dialing the same number.

HKR, Country\XXXX, BLACKLISTING, 1, \

01,\		;BlackListing Method for NoDialTone

01,\		;BlackListing Method for KeyAbort

01,\		;BlackListing Method for Busy

01,\		;BlackListing Method for NoConnection

01,\		;BlackListing Method for NoAnswer

01,\		;BlackListing Method for VoiceAnswer

3C,00,\		;BlackListingDelay1

05,00,\		;BlackListingDelay2

00,00,\		;BlackListingDelay3

00,00,\		;BlackListingDelay4

00,00,\		;BlackListingCount1

00,00,\		;BlackListingCount2

02		;BlackListing Control Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

#### Switzerland

After 6 consecutive failed attempt, number is permanently blacklisted until manual intervention.

HKR, Country\XXXX, BLACKLISTING, 1, \

03,\		;BlackListing Method for NoDialTone

03,\		;BlackListing Method for KeyAbort

03,\		;BlackListing Method for Busy

03,\		;BlackListing Method for NoConnection

03,\		;BlackListing Method for NoAnswer

03,\		;BlackListing Method for VoiceAnswer

05,00,\		;BlackListingDelay1

05,00,\		;BlackListingDelay2

00,00,\		;BlackListingDelay3

00,00,\		;BlackListingDelay4

00,00,\		;BlackListingCount1

06,00,\		;BlackListingCount2

03		;BlackListing Control Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

#### Non Blacklisting Countries

HKR, Country\XXXX, BLACKLISTING, 1, \

00,\		;BlackListing Method for NoDialTone

00,\		;BlackListing Method for KeyAbort

00,\		;BlackListing Method for Busy

00,\		;BlackListing Method for NoConnection

00,\		;BlackListing Method for NoAnswer

00,\		;BlackListing Method for VoiceAnswer

00,00,\		;BlackListingDelay1

00,00,\		;BlackListingDelay2

00,00,\		;BlackListingDelay3

00,00,\		;BlackListingDelay4

00,00,\		;BlackListingCount1

00,00,\		;BlackListingCount2

00		;BlackListing Control Flags

bit0	-	Enable permanent blacklisting

bit1	-	Enable resetting of a blacklist number upon a

successful attempt

bit2	-	Enable distinct failure condition

bit3	-	reserved

bit4	-	reserved

bit5	-	reserved

bit6	-	reserved

## CALLERID

- Caller ID Parameter

- Defined Values

1 Byte	Defines Caller ID type.  The table below shows the currently defined types.  More types will be added in the future.

| ID Type | Country |
| --- | --- |
| 0 | No Caller ID |
| 1 | United States |
| 2 | Japan |
| 3 | Japan (Unimodem) |
| 4 | French |
| 5 | UK |

## SPEEDADJUST

- Initial Speed Selection Adjustment Parameters

- Defined Values For HSF

1 Byte	V90 speed adjust index. The index is actually a real number represented in a mantissa + exponent format where the 4 most significant bits (L) define the mantissa and the 4 least significant bits (R) define the exponent in negative form. This number limits the percentage of BLER allowed:
 BLER limit = L*10^(-R) % 
The higher the maximal allowed BLER is, the more aggressive the connectivity becomes, i.e. the connection rates tend to be higher. Typically, decreasing the exponent by 1 results in one rate higher connections. Note, that the above is very line dependent.
Examples:
Byte 41 defines the limit 0.4% (or 0.004)
Byte f1 defines the limit 1.5%
Byte f0 defines the limit 15%
The value of 0 indicates the default setting of the modulation, which may change from time to time.

1 Byte				K56 speed adjust index. Not supported.

1 Byte				V.34 speed adjust index. Not supported.

## FLAGS

- Four bit-mapped bytes are used as boolean flags for specifying certain modem behaviors.

- Defined Values (Flag0)

bit 0	Enable use of Alternate filter instead of the Primary filter for dial tone detection.

bit 1	Enable use of Alternate filter instead of the Primary filter for call progress detection.

bit 2	Enable use of Alternate filter instead of the Primary filter for secondary dial tone detection.

bit 3	Enable use of Alternate filter instead of the Primary filter for secondary dial tone detection after dialing the international access code

bit 4	Unused

bit 5	Unused

bit 6	Unused

bit 7	Unused

- Defined Values (Flag1)

bit 0	Enable enforcement of  Dial Tone detection. Setting of this flag prevents blind dialing and overrides the X0, X1, and X3 commands.

bit 1 	Enable enforcement of Data calling tone or CI. Setting of this flag prevents the disabling of calling tone transmission.

bit 2	Enable enforcement of 1800 Hz guard tone.  Setting of this flag forces the transmission of guard tone.

bit 3	Enable enforcement of Busy detection. Setting of this flag forces the detection of BUSY and overrides the X0, X1, and X2 commands.

bit 4	Enable the use of S7 as the delay when W command is executed.

bit 5	Enable the Hang Up of call with Call Waiting is detected.

bit 6	Unused

bit 7	Unused

- Defined Values (Flag2)

bit 0	Disable the execution of  ATH1.

bit 1	Disable the execution of ATDTxxPyy (Disallow dialing one dial string by a mix of tone and pulse dialing.

bit 2	Enable the use of pulse dialing (ATDP).

bit 3	Enable the dialing of DTMF A, B, C and D digits.

bit 4	Enable Blacklisting.

bit 5	Enable Long tone is busy. Causes the modem to report BUSY when continuous dial tone is detected after call establishment

bit 6	Enable selection of U Law

bit 7	Disable Bell modes (Bell 103 mainly since Bell212=V.22)

- Defined Values (Flag3)

bit 0				Enable to ignore commas before the first dialed digit

bit 1				Enable using the DTMF_COMP_LEVEL parameters in countries with

complex impedance.

bit 2				Unused

bit 3				Unused

bit 4				Unused

bit 5				Unused

bit 6				Unused

bit 7	Unused

# OEM Parameter Values

- Location of Key in WINDOWS 95 Registry is

- HKEY_LOCAL_MACHINE \ System \ CurrentControlSet \ Services \ Class \ Modem \ [XXXX] \ OEM

[XXXX]		-	PnP modem key number

## ATI Subkey Values

### ATI0

CHAR[]	Reports product code

### ATI1

CHAR[]	Reports Pre-Computed checksum

### ATI3

CHAR[]	Reports Firmware Version

### ATI4

CHAR[]	Reports OEM defined string

### ATI6

CHAR[]	Reports data pump model and internal code revision

### ATI7

CHAR[]	Reserved

## ATPLUSID Subkey Values

### AT+GCAP

CHAR[]			Causes the modem to transmit lines of information text listing additional capabilities

commands +<name>s, which is intended to permit the user to identify the overall

capabilities of the modem.

### AT+GMI

CHAR[]			Reports the modem product manufacturer provided in the product configuration file

### AT+GMM

CHAR[]			Reports the modem product model provided in the product configuration file

### AT+GMR

CHAR[]			Reports the modem version, revision level or date as provided in the product

configuration file

### AT+GOI

CHAR[]			Reports device identification based on the ISO system for registering unique

object identifiers.

### AT+GSN

CHAR[]			Reports the modem product modem serial number provided in the product

configuration file.

### AT+FMI

CHAR[]			Reports the modem product manufacturer provided in the product configuration file

### AT+FMM

CHAR[]			Reports the modem product model provided in the product configuration file

### AT+FMR

CHAR[]			Reports the modem version, revision level or date as provided in the product

configuration file

## SREGS

- Used to set S-Register default values

- Defined Values

1 Byte		Defines S0 - Number of rings till auto answer

1 Byte		Defines S1 - Ring counter

1 Byte		Defines S2 - Escape Character

1 Byte		Defines S3 - Carriage Return Character

1 Byte		Defines S4 - Line Feed Character

1 Byte		Defines S5 - Backspace Character

1 Byte		Defines S6 - Wait time(ms) for dial tone before blind dialing

1 Byte		Defines S7 - Wait time(me) for Carrier

1 Byte		Defines S8 - Defines pause time in seconds for “,”

1 Byte		Defines S9 - Defines carrier detect response time (units in 0.01sec)

1 Byte		Defines S10 - Defines carrier loss disconnect time (units in 0.01sec)

1 Byte		Defines S11 - DTMF tone duration

1 Byte		Defines S12 - Escape prompt delay

1 Byte		Defines S13 - Reserved

1 Byte		Defines S14 - General Bit Mapped Option

1 Byte		Defines S15 - Reserved

1 Byte		Defines S16 - Reserved

1 Byte		Defines S17 - Reserved

1 Byte		Defines S18 - Reserved

1 Byte		Defines S19 - Reserved

1 Byte		Defines S20 - Reserved

1 Byte		Defines S21 - General Bit Mapped Option

1 Byte		Defines S22 - General Bit Mapped Options

1 Byte		Defines S23 - General Bit Mapped Options

1 Byte		Defines S24 - Sleep Inactivity Timer (units of 10s)

1 Byte		Defines S25 - Length of time that ignores DTR before hanging up

1 Byte		Defines S26 - RTS to CTS delay

1 Byte		Defines S27 - General Bit Mapped Option

1 Byte		Defines S28 - General Bit Mapped Option

1 Byte		Defines S29 - Flash delay (units of 10ms)

1 Byte		Defines S30 - Disconnect inactivity timer (units in minutes)

1 Byte		Defines S31 - General Bit Mapped Option

1 Byte		Defines S32 - XON character

1 Byte		Defines S33 - XOFF character

1 Byte		Defines S34 - Reserved

1 Byte		Defines S35 - Reserved

1 Byte		Defines S36 - LAPM failure control

1 Byte		Defines S37 - DCE connection speed

1 Byte		Defines S38 - Delay before forced hangup(error correction)

1 Byte		Defines S39 - Flow control

## AT+MSSTR

CHAR[]	Defines default modulation string

## AT+MS

4 Bytes	Defines default modulation type

4 Bytes	Enables/Disables automatic modulation negotiation.

4 Bytes	Defines the lower limit in the transmit direction.

4 Bytes	Defines the upper limit in the transmit direction.

4 Bytes	Defines the lower limit in the receive direction.

4 Bytes	Defines the upper limit in the receive direction.

## V25TER

- Used to set default operation of modem

- Defined Values

### +IPR

4 Bytes	Defines fixed DTE rate

### +ILRR

4 Bytes	Defines DTE rate reporting

### +MR

4 Bytes	Defines modulation reporting

### +ESR

4 Bytes	Defines the use of the selective repeat(SREF) option in V.42.

### +EFCS

4 Bytes	Defines the use of the 16-bit or 32-bit frame check sequence (FCS) option in

V.42.

### +ER

4 Bytes	Defines error correction reporting.

### +DR

4 Bytes	Defines data compression reporting

### A8E Related Parameters

1 Byte	Defines A8E allowed call function value

1 Byte	Reserved use

### +A8E Parameters

4 Bytes	Defines an enable/disable of +A8x indications during modem-controlled

V.8 origination negotiation.

4 Bytes	Defines an enable/disable of +A8x indications during modem-controlled

V.8 answer negotiation.

4 Bytes	Defines the V.8 CI signal call function (octet)

4 Bytes	Defines an enable/disable of V.8bis negotiations.

20 Bytes	Defines ‘Cfrange’. Used when DCE-controlled negotiations with +A8x

indications are set to enable.

20 Bytes	Defines ‘Protrange’. Used when DCE-controlled negotiations with +A8x

indications are set to enable.

### +DS Parameters (Data Compression)

4 Bytes	Defines the desired direction(s) of operation of the data compression

function from the DTE point of view.

4 Bytes	Defines whether or not the modem should continue to operate if the desired

data compression negotiation result is not obtained.

4 Bytes	Defines, in regards to data compression, the maximum number of dictionary

entries which should be negotiated.

4 Bytes	Defines, in regards to data compression, the maximum string length to be

negotiated.

### +EB Parameters (Break Handling in Error Control Operation)

4 Bytes	Defines whether the break is to be ignored, i.e., not signaled to remote DCE.

4 Bytes	Defines length of time of any V.42 L-SIGNAL  before break.

4 Bytes	Defines break length.

### +ES Parameters (Error control and Synchronous Mode Selection)

4 Bytes	Specifies the initial requested mode of operation when the modem is

operating as the originator.

4 Bytes	Specifies the acceptable fallback mode of operation when the modem is

operating as the originator.

4 Bytes	Specifies the acceptable fall back mode of operation when the modem is

operating as the answer or specifies V.80 Synchronous Access Mode.

### +ESA Parameters (Synchronous Access Mode)

4 Bytes	Specifies the bit sequence transmitted by the modem while a transmit data buffer

underrun condition occurs, while operating in Transparent sub-Mode.

4 Bytes	Specifies the bit sequence transmitted by the modem when a transmit date buffer

underrun condition occurs immediately after a flag, while operating in Framed

sub-Mode.

4 Bytes	Specifies the action taken by the modem when a transmit data buffer underrun or

overrun condition occurs immediately after a non-flag octet, while operating in

Framed sub-Mode.

4 Bytes	Specifies the CRC polynomial used while operating in Framed sub-Mode.

4 Bytes	Specifies if Non Return to Zero Inverted (NRZI) encoding is to be used by the

modem for transmit and receive data.

4 Bytes	Specifies the an octet value to be used while performing character-oriented

framing.

### +ETBM Parameters (Call Termination Buffer Management)

4 Bytes	Specifies length of time, after the local DTE requests call disconnection,  that all

buffered transmit data will be discarded.

4 Bytes	Specifies length of time, after the local DTE requests call disconnection,  that all

buffered receive data will be discarded.

4 Bytes	Specifies length of time that the modem will attempt to continue to deliver the

before abandoning the attempt and discarding the data.

### +FIT (DTE Inactivity Timeout)

4 Bytes	Specifies the DTE inactivity timeout in seconds.

4 Bytes	Specifies the inactivity action type.

### +ICF  (Char Format Options)

4 Bytes	Specifies the char format.

4 Bytes	Specifies the char parity.

### +IFC (DTE Modem Local Flow Control)

4 Bytes	Specifies the method used by the DTE to control the flow of received data from

the modem.

4 Bytes	Specifies the method to be used by the modem to control the flow of transmitted

data from the DTE.

### +ITF (Transmit Flow Control Thresholds)

4 Bytes	Specifies the threshold, in octet, above which the modem will generate a flow off

signal. Applicable in Synchronous Access Mode.

4 Bytes	Specifies the threshold, in octet, below which the modem will generate a flow on

signal. Applicable in Synchronous Access Mode.

4 Bytes	Species the number of bytes in the transmit buffer.

### MISC AT Commands

1 Byte	Defines E	-	Command echo

1 Byte	Defines Q	-	Result code suppression

1 Byte	Defines V	-	Modem response format

1 Byte	Defines T	-	Select tone dial

1 Byte	Defines &D	-	DTR behavior

1 Byte	Defines &C	-	RLSD behavior

1 Byte	Defines L	-	Monitor speaker loudness

1 Byte	Defines M	-	Monitor speaker mode

1 Byte	Defines X	-	Result code selection and call progress monitoring

control.

1 Byte	Defines &T	-	Request for RDL

1 Byte	Defines %E	-	Enable/Disable line quality monitor, auto-retrain, and auto

rate renegotiation.

1 Byte	Defines &G	-	Select guard tone.

1 Byte	Defines &P	-	Select pulse dial Make/Break ratio.

1 Byte	Defines &R	-	Defines RTS option.

## CONSTTONEPARAMS

Tone Detect Parameters used by the HSF Data Pump

Defined Values

24 Bytes*			Defines Tone ATBel

24 Bytes*			Defines Tone AtV25

24 Bytes*			Defines Tone CI Center Frequency

24 Bytes*			Defines Tone CI Side Band

24 Bytes*			Defines ToneC 2912

24 Bytes*			Defines ToneB 2312

24 Bytes*			Defines ToneB 2750

24 Bytes*			Defines ToneB 2130

24 Bytes*			Defines ToneA Not ST

24 Bytes*			Defines ToneC 1650

24 Bytes*			Defines Tone 1270

24 Bytes*			Defines Tone CReSEG1 2002

*Parameter declared in the following format:

2 Bytes	Defines the Center Frequency of the primary tone detector. Value represents the frequency in Hz.

2 Bytes	Defines the Bandwidth of the primary tone detector. Value represents a range in Hz (with the Center Frequency in the middle. Tones within the specified range should be detected (e.g. if Center Frequency is 450 and the Bandwidth is 400, tones in the range of 250-650Hz are detected).

4 Bytes	Defines primary filter Response Time. Value reflects the minimum time in milliseconds that takes the detector to detect a tone. Longer response time provides a more accurate tone detection. Normally, the default value for this parameter should not be changed.

2 Bytes	Defines primary THRESHU. Value represents an upper limit level in dBm. If the

tone level that is being detected  exceeds the value of THRESHU, then the tone detect flag is SET. This value will be overwritten by the values in the THRESHOLD country key. The value should be greater than the Response Time.

2 Bytes	Defines primary THRESHL. Value represents a lower limit level in dBm. If the tone

level falls below THRESHL then the tone detect flag is RESET.  This value will be overwritten by a value derived from the values in the THRESHOLD country key. This value should be greater than the Response Time.

4 Bytes	Reserved.

4 Bytes	Defines primary tone ON stability time. Value reflects the minimum time in milliseconds for which the modem must detect continuous tone in order for the “Tone On” event to be reported.

4 Bytes	Defines primary tone OFF stability time. Value reflects the minimum time in milliseconds for which the modem must detect continuous tone non-existence in order for the “Tone Off” event to be reported.

## AT_MISC_DEF

- Misc AT Command Definitions

4 Bytes				Defines ATW Command

4 Bytes				Defines AT+FAE Command

4 Bytes				Enable V90

4 Bytes				RESERVED (Default = 0x0000)

## MISC_VOICE_PARMS

- Misc Voice Parameter Definitions

4 Bytes				Defines DSVD +VGS minimum value

4 Bytes				Defines DSVD +VGS maximum value

4 Bytes				Defines DSVD +VGS default value

4 Bytes				Defines DSVD +VGM minimum value

4 Bytes				Defines DSVD +VGM maximum value

4 Bytes				Defines DSVD +VGM default value

4 Bytes				Defines FDSP +VGS minimum value

4 Bytes				Defines FDSP +VGS maximum value

4 Bytes				Defines FDSP +VGS default value

4 Bytes				Defines FDSP +VGM minimum value

4 Bytes				Defines FDSP +VGM maximum value

4 Bytes				Defines FDSP +VGM default value

1 Byte				Defines FDSP +VGR minimum value

1 Byte				Defines FDSP +VGR maximum value

1 Byte				Defines FDSP +VGR default value

1 Byte				Defines TAM +VGR minimum value

1 Byte				Defines TAM +VGR maximum value

1 Byte				Defines TAM +VGR default value

1 Byte				Defines the Voice TX Level lower limit.

1 Byte		 		Defines the Voice Telout limit.

1 Byte		 		Defines the Voice Line Rx Gain Bias in dB units.

1 Byte		 		Defines the SpeakerPhone Line Rx Gain Bias in dB units.

1 Byte		 		Defines the SpeakerPhone Mic Rx Gain Bias in dB units.

1 Byte				Defines the Voice Codec Mic gain in dB units.

## CADENCE

- OEM Tone Cadence Parameters

- Defined Values

1 Byte				Defines number of phases (1,2). Value reflects number of different Phases (pulse)

that identifies a tone’s cycle.

36 Bytes*			Defines cadence for Fax Calling tone cadence

4 Bytes				Defines a special Fax Calling tone timer

*Parameter declared in the following format:

Minimum ON time (ms);  Phase 1			-	4 Bytes

Maximum ON time (ms); Phase 1			-	4 Bytes

Minimum OFF time (ms); Phase 1			-	4 Bytes

Maximum OFF time (ms); Phase 1			-	4 Bytes

Minimum ON time (ms);  Phase 2 (if used)		-	4 Bytes

Maximum ON time (ms); Phase 2 (if used)		-	4 Bytes

Minimum OFF time (ms); Phase 2 (if used)		-	4 Bytes

Maximum OFF time (ms); Phase 2 (if used)		-	4 Bytes

Number of cycles needed for detection (if used)		-	4 Bytes

## AUDIO_AEC

- DSVD Digital FDSP Audio Parameters that are used to tune DSP.

- Defined Values

2 Bytes	DP_TIME_CLEAR_OUT_SPK_BIT

2 Bytes	DP_LINE_RECEIVE_INT_SCL_FCT

2 Bytes	DP_LINE_RECEIVE_FRC_SCL_FCT

2 Bytes	DP_LINE_TRANSMIT_INT_SCL_FCT

2 Bytes	DP_LINE_TRANSMIT_FRC_SCL_FCT

2 Bytes	DP_MIC_INT_SCL_FCT

2 Bytes	DP_MIC_FRC_SCL_FCT

2 Bytes	DP_SPK_INT_SCL_FCT

2 Bytes	DP_SPK_FRC_SCL_FCT

2 Bytes	DP_HANDSET_DOUBLE_TX_LIMIT

2 Bytes	DP_HANDSET_RXMODE_TX_LIMIT

2 Bytes	DP_HANDSET_SILENT_TX_LIMIT

2 Bytes	DP_HANDSET_NEG56DB

2 Bytes	DP_HANDSET_MIC_INT_SCL_FCT

2 Bytes	DP_HANDSET_MIC_FRC_SCL_FCT

2 Bytes	DP_HANDSET_SPK_INT_SCL_FCT

2 Bytes	DP_HANDSET_SPK_FRC_SCL_FCT

2 Bytes	DP_HEADSET_MIC_INT_SCL_FCT

2 Bytes	DP_HEADSET_MIC_FRC_SCL_FCT

2 Bytes	DP_HEADSET_SPK_INT_SCL_FCT

2 Bytes	DP_HEADSET_SPK_FRC_SCL_FCT

2 Bytes	DP_DTD_THRESHOLD

2 Bytes	 			DP_NFREF

## MOD_THRESHOLD

Defines the threshold for a given modulation.

- For a given modulation of 4 Bytes

-	Byte 0 to 1 defines the Originate Threshold

-	Byte 2 to 3 defines the Answer Threshold

- Defined Values

4 Bytes				V21

4 Bytes				V21Fax

4 Bytes				V23

4 Bytes				V22

4 Bytes				V22b

4 Bytes				V32

4 Bytes				V32b

4 Bytes				V34

4 Bytes				K56

4 Bytes				V90

4 Bytes				V27

4 Bytes				V29

4 Bytes				V17

## CALLWAITING_PARMS (TWO-LINE only)

- Call Waiting Threshold Parameters

- Defined Values

10 Bytes*			Defines detection parameter for V90 Modulation

10 Bytes*			Defines detection parameter for K56 Modulation

10 Bytes*			Defines detection parameter for V34 Modulation

10 Bytes*			Defines detection parameter for V32 Modulation

10 Bytes*			Defines detection parameter for V32B Modulation

*Parameter declared in the following format:

Upper bound threshold				-	2 Bytes

Lower bound threshold				-	2 Bytes

Alternate Upper bound threshold			-	2 Bytes

Alternate Lower bound threshold			-	2 Bytes

Receive power level indicator 			-	2 Bytes

## AT_MISC_CELL_DEF (WIRELESS only)

- Misc WIRELESS AT Command Definitions

4 Bytes				Defines AT+WS46 Command

## FLAGS

- Defined Values (Flag0)

bit 0				Reserved

bit 1				Enable notification of use of an analog speaker

bit 2				Enable Data/Fax remote TAM

bit 3				Enable Data/Fax Voice View

bit 4				Enable LCS present

bit 5				Enable the use of 3.3V Analog IA

bit 6				Enable the detection of remote hang up

bit 7	Enable speaker mute hardware present

- Defined Values (Flag1)

bit 0				Enable country detection from hardware

bit 1				Enable validation of ACF AT command set (S46, %C, \K, &K, &Q)

bit 2				Enable PME

bit 3				PME with interrupt. When set modem will assert PME and generate

interrupt upon wake up from D3.

bit 4				Enable to allow a NONE response in country select tab

bit 5				Control speaker mute polarity. ‘1’ implies active HIGH and ‘0’ implies active LOW

bit 6				Enables speaker boost. ‘1’ will set IA speaker output to maximum level. ‘0’ will

use default value.

bit 7	Enable the allowing of +VGM and +VGS to control microphone and speaker. If

value is set to ‘0’, driver will use DSVD default values.

- Defined Values (Flag2)

bit 0				Reserved

bit 1				Disable the forcing of LAPM when V.8bis fails in V90.

bit 2				Disables DOS support by unchecking the box

bit 3				Enables the user to select whether  DOS is supported or not (via DOS select box)

bit 4				Enable special local handset handling based on +VCID setting.

bit 5				Enable the detection by the hardware of an extension off hook event.

bit 6				Enable the customization of driver for LAN Modem.

bit 7	Enable the maintaining of the line connection on a port close.

- Defined Values (Flag3)

bit 0				Enable access to all GPIO bits.

bit 1				Enable DAA support of D3 Caller ID circuitry

bit 2				Enable “Line in Use” feature. Will check to see if handset is down before dialing. If

handset is up,  a BUSY is reported. LCS Present Flag must be enabled.

bit 3				Disable the limiting of the V.42 data rate to the PC

bit 4				Reserved

bit 5				Enable the disallowing of answer in V23 mode.

bit 6				Reserved

bit 7	Reserved

## PROPERTIES

- NVRam Capabilities

4 Bytes				Defines NVRam Capabilities

## DaaType

Used to define DAA type being used.

| Defined Value | DaaType |
| --- | --- |
| 0 | Standard DAA |
| 1 | Smart DAA |
| 2 | Soft DAA |

## SmartDAAParams (Smart DAA Only)

- Smart DAA Parameters

- Defined Values

4 Bytes			Defines the R6 value of the circuit in K ohms

4 Bytes			Defines the R26 value of the circuit in K ohms

4 Bytes			Defines the R28 value of the circuit in ohms

4 Bytes			Defines the R56 value of the circuit in K ohms for Caesar1 (20434-X4)

4 Bytes			Defines the R30 value of the circuit in K ohms for Caesar2 (20463-X2)	4 Bytes			Defines the R38 value of the circuit in ohms

4 Bytes			Defines the power limit on the AC transistor in mW

4 Bytes			Defines the power limit on the DC transistor in mW

4 Bytes			**Impedance values to be used.

5 Bytes			*Defines the 5 DC Loop voltage points of Curve 0 (units of 0.5 volts)

5 Bytes			*Defines the 5 DC Loop voltage points of Curve 1 (units of 0.5 volts)

5 Bytes			*Defines the 5 DC Loop voltage points of Curve 2 (units of 0.5 volts)

5 Bytes			*Defines the 5 DC Loop voltage points of Curve 3 (units of 0.5 volts)

5 Bytes			*Defines the 5 DC Loop voltage points of Curve 4 (units of 0.5 volts)

5 Bytes			*Defines the 5 DC Loop current points of Curve 0 (units of mA)

5 Bytes			*Defines the 5 DC Loop current points of Curve 1 (units of mA)

5 Bytes			*Defines the 5 DC Loop current points of Curve 2 (units of mA)

5 Bytes			*Defines the 5 DC Loop current points of Curve 3 (units of mA)

5 Bytes			*Defines the 5 DC Loop current points of Curve 4 (units of mA)

5 Bytes			*Defines the 5 init DAC values (one for each DC loop)

4 Bytes			Flags

* The order of the defined points are from point 1 to point 5 where point 1 is the LSB BYTE and point 5 is the MSB Byte as defined in the MS REGISTRY format specification. Value is used in association with the RELAY values defined in the County section.

** This value defines 4 impedance values which are selectable based on the values defined in RELAYS of the Country section. Impedance value 1 is the LSB Byte and Impedance value 4 is the MSB Byte as defined in the MS REGISTRY format specification.

### SmartDAA Flags

The 4 bytes of the Flags field correspond to System Side registers as specified in the following table. The description in the table is digit (nibble) by digit, digit #1 being the leftmost character in the Flags line, and digit #8 is the rightmost one.

| Digit # | SSD Reg# | Bits | Name |
| --- | --- | --- | --- |
| 1 | N/A |  |  |
| 2 | N/A |  |  |
| 3 | N/A |  |  |
| 4 | N/A |  |  |
| 5 | 0B | [7:5] | DUTY_CYCLE for ultra low power |
| 6 | 0B | [3:0] | DUTY_CYCLE for low power |
| 7 | 06 | [2:0] | CLKSHIGH (4MHz high power) |
| 8 | 06 | [6:4] | PWROUT |

DUTY_CYCLE for ultra low power

digit	%duty cycle

0	1

2	2

4	3

6	4

8	5

A	6

C	7

E	8

DUTY_CYCLE for low power

digit	%duty cycle

0	1

1	2

2	3

3	4

4	5

5	6

6	7

7	8

8	9

9	10

A	11

B	12

C	13

D	14

E	15

F	16

CLKSHIGH (4MHz high power)

digit	%duty cycle

0	50

1	14.3

2	28.3

3	42.9

7	50

## SPKR_MUTE_DELAY

- Speakers Mute Delay

2 Bytes				Defines the Speakers Mute Delay

## DTMF_COMP_LEVEL

4 bytes*			DTMF HightBand power compensation values (for countries with complex impedance).

4 bytes*			DTMF LowBand power compensation values (for countries with complex impedance).

*Parameter declared in the following format:

Each byte defines one compensation value (4 bytes for 4 different bands).

## HwData

DWORD value. Defines various HW options.

| Bits | Name | Comment |
| --- | --- | --- |
| [0] | Voice codec | 0 – Voice codec not present 1 – Voice codec present |
| [1-3] | Microphone Gain | 0 – 0 dB 1 – 10 dB 2 – 15 dB 3 – 20 dB |
| [4] | Microphone Attenuation | 0 – Disable 4 dB microphone input attenuation 1 – Enable 4 dB microphone input attenuation |
| [5-7] | DAC Input Source | 0 – DAC interpolator bypass 1 – Interpolator bits [22-37] 2 – Interpolator bits [21-36] 3 – Interpolator bits [20-35] |
| [8] | Modem Speaker Input | 0 – Speaker input is Tx filter of the codec 1 – Speaker input is line in. |
| [9] | Oscillator Mode When Modem Is Idle | 0 – Keep oscillator on when modem is idle 1 – Keep oscillator off when modem is idle |
| [10] | Riptide Board | 0 – Not a Riptide board 1 – A Riptide board |
| [11] | Wake On Ring When COM Port Is Closed | 0 – Disable wake-on-ring when port is closed 1 – Enable wake-on-ring when port is closed |
| [12] | Enable Interrupts During Modem Interrupt | 0 – Interrupts are disabled during modem’s ISR. 1 – Interrupts are enabled during modem’s ISR. |
| [13] | Charlemagne Handset | 0 – Charlemagne handset not present 1 – Charlemagne handset present |
| [14] | Off Hook Control In PDC Mode | 0 – Set ADP GPIO high to go "off hook" in PDC mode 1 – Set ADP GPIO low to go "off hook" in PDC mode |
| [15] | PME with FET | 0 – GPIO14 is unconnected . No FET. 1 – a FET circuit exists between PME# of PCI bus and PME# of R6793-12. The FET is meant to gate the PCI PME# signal. The FET is controlled by GPIO14 (RESETN bit). PME# is enabled by the driver only upon going to D3 state (StandBy). |
| [16-31] | Reserved | Set to 0 |

# Top Level Modem Parameters Values

- Location of Key in Windows 95 Registry is

- HKEY_LOCAL_MACHINE \ System \ CurrentControlSet \ Services \ Class \ Modem \ [XXXX]

[XXXX]		-	PnP modem key number

## Buffers

CHAR[]	Defines if hardware buffers are used to handle the PCI bus latency problem. Possible values - “ON” or “OFF”.

## BuffersDelay

BYTE	Defines the interrupt latency that the modem can handle in terms of buffer number ( this will later be changed to be in term of time). Default is value 03 if buffers field is “ON”, it is advised to change the BuffersDelay to 05.

## BondOption

CHAR[]	Defines the functionality supported by the modem driver. The string value should be a hexadecimal bit mask number. The Bit Mask is translated as follows:

| Bit | Enables |
| --- | --- |
| 0 | V34 |
| 1 | Full duplex V34, VFC, V32, V22 |
| 2 | 28Mhz/22Mhz |
| 3 | V34Fax |
| 4 | Voice |
| 5 | SpeakerPhone |
| 6 | VFC |
| 7 | K56flex |
| 8 - 15 | Not Assigned |