## In Search Of A New DRAM Solution

The DRAM market of the 1970s and '80s - nearly unified around a single, generic DRAM - is fragmenting in the 1990s. Standard DRAMs are being offered in alternative pinouts, varying feature sets, and timing variations. In addition, a spate of "new" DRAMs have also appeared, and the proliferation continues because these new DRAMs have not satisfied a critical need.

Today's standard DRAMs employ an interface and memory architecture optimized for system architectures of the early 1970s. Though no one questions that these system architectures are archaic, the market has continued to adapt the same DRAM architecture in the belief that only these memories offer the smallest die, and, therefore, the lowest cost. Today's variations on the DRAM theme do nothing to challenge that assumption, either. Many have larger die, are more complex, and do cost more to manufacture.

Many new and high-volume applications are characterized by several independent processes sharing a common memory. In such cases, controllers will multiplex each process' access, giving each a turn to read or write in fast, short, memory bursts. The problem is, each process is typically looking for data in different "rows," and conventional DRAMs and their new DRAM derivatives cannot accommodate the processes without repeated, lengthy, row-access "latencies." Under these circumstances, DRAM innovations that deliver relatively high peak bandwidth are burdened by a high percentage of full, row-access, delays. The upshot is that useful bandwidth (i.e. the percentage of total time when reading or writing is taking place) is severely diminished.

MoSys believes that high-capacity DRAM memory must adapt architecturally to system need rather than vice-versa. As such, MoSys developed a new DRAM architecture that meets the following criteria:

- Organized the way systems use memory;

- Leads rather than lags system bandwidth demand;

- Features an uncomplicated interface;

Steamlined in features and functions for lowest cost.

Table 1. Performance of New Memories

| Type | Total Latency | Column Cycle | Column Bandwidth | Net Bandwidth | Bus Efficiency |
| --- | --- | --- | --- | --- | --- |
| Std DRAM | 80 ns | 35 ns | 114 MB/s | 87 MB/s | 81% |
| EDO DRAM | 100 ns | 25 ns | 160 MB/s | 105 MB/s | 66% |
| SDRAM-66 MHz | 102 ns | 15 ns | 267 MB/s | 166 MB/s | 62% |
| SDRAM-100 MHz | 75 ns | 10 ns | 400 MB/s | 253 MB/s | 63% |
| RDRAM | 108 ns | 8 ns | 500 MB/s | 206 MB/s | 41% |
| MDRAM | 20ns | 6 ns | 660 MB/s | 490 MB/s | 74% |

## The New "Old" DRAMs

Every PC and workstation designer knows that standard DRAMs fail to keep up with the performance needs of their graphics subsystems. Even video-RAM has failed to fill this gap because of its inherent price disadvantage and difficulties in scaling beyond 4 Mbits.

This problem was seen as an opportunity by DRAM companies. Some offered conventional DRAM extensions, such as wide, fast, and extended-data-out (EDO) versions - all incremental modifications to existing DRAMs. Others offered evolutionary alternatives, such as synchronous DRAM (SDRAM) and synchronous graphics RAM (SGRAM). Here, they've added a contemporary interface onto the classical RAM architecture. Windows RAM (WRAM) is simply a recasting of VRAM, which is itself based on classic DRAM architecture. And, the Rambus DRAM (RDRAM) revolutionary alternative defines a new interface concept mated to standard DRAM. All of these alternatives modify the interface of the DRAM while retaining its fundamental architecture.

Each of these memories do solve one or more graphics memory problems. However, none has simultaneously solved the performance, cost, and complexity problems.

One can sum up the shortcomings of the new "old" memories as follows:

- Low net bandwidth - none of these memories provide a net bandwidth that is close to peak bandwidth when multiplexing short bursts for multiple processes.

- Bandwidth that lags system requirements - these memories lack the ability to keep pace with systems' insatiable memory bandwidth requirements.

- Poor efficiency - memory bus efficiency (net bandwidth/peak bandwidth) is generally poor, hence, design-in is more costly and more difficult. High pin count, complicated interfaces, and/or very high clock rates are attempts to compensate.

- Added circuitry; added costs -compared to standard asynchronous DRAM of equal bit density, many of the new DRAMs are more complex, have larger die sizes, and, therefore, inherently more costly to produce.

- Coarse granularity -compared to earlier generations, the new DRAMs provide less memory granularity, forcing designers to buy more capacity than needed in order to get the desired bandwidth.

In sum, though all the new memories solve performance problems to some degree, unfortunately, they all create additional problems whose scope and degree are directly proportional to the memory's potential performance.

Multiplexing And The Latency Problem

Figure 1. Graphics Memory Bottleneck

Graphics subsystems, like high-definition television (HDTV), set-top converters, games, networking hubs, and networking routers can be characterized by several independent processes sharing some common memory. In the case of the graphics subsystem, these processes may be the video controller, system interface, bit-block transfer (BitBLT) engine, and display controller.

To accommodate these independent processes, each is given round-robin access to memory for fast, short, memory read or write bursts. However, when each successive access has a different memory row address, each access will first have to transfer data from the memory array to the sense amps, adding substantial latency. Thus, the proportion of time that the memory is actually reading or writing shrinks with the increased frequency of row-access operations. As more processes are added, net (useful) bandwidth decreases further.

A possible solution is longer transfers.  If the burst transfer size is increased for each read or write operation, then net bandwidth will also increase because useful operations occur longer while latencies remain the same. There is a limit to how large the transfer size can be made, though, because the larger transfer size reduces the multiplex switching rate causing the queued processes to wait longer for their chance at memory access. As a result, designers make difficult tradeoff choices between bandwidth, buffer sizes and transfer lengths.

## Solving The Latency Problem,  And Others, Too

A DRAM architecture that can reduce both the frequency and duration of row-access operations will allow greater net bandwidth at smaller transfer sizes. As a result, designers need to worry less about trading off bandwidth against buffer size and transfer length. They can choose smaller buffers and transfer lengths and still achieve high net bandwidth.

Imagine an array of very fast 256 Kbit DRAMs, organized as 32KByte memories, each little DRAM fitted with 32-bit data I/O, and each attached to a common memory bus. Now shrink the model down until the array of 256 Kbit DRAMs and their memory bus will fit onto a monolithic silicon chip. That is essentially the new DRAM architecture developed by MoSys, Incorporated.

Called Multibank DRAM, or MDRAM, the multiple, independent, 32 KByte memories (called banks) that characterize this architecture are a giant-step forward in solving the latency problem just described.

The banks are independent, and 1 MByte of memory contains 32 of them. Any number of banks can be active, at the same time, on the same device. And each active bank can have a different row address selected. As processes are multiplexed, successive accesses are frequently to different banks, but typically the required rows in those banks are already active -- so the latencies involved are much shorter. And, if an access is to a row address that is not active, the row-access operation can occur in parallel with a read or write operation. Thus, part of its latency duration is hidden during another useful operation.

To illustrate how effectively the MDRAM architecture can increase net bandwidth, consider the probability of a low-latency "column hit" for a dual-bank SDRAM being used by four independent processes. The probability, P, is 100 (1-1/B)N-1 where B is the number of banks and N is the number of processes. For the dual-bank chip and four competing process situation, the probability of a column hit is only 12.5 percent, and the likelihood of a miss is 87.5 percent. In the case of a miss, the peak bandwidth is burdened by a full row-access latency, and that will happen 87.5 percent of the time. Making the same calculation for a 32-bank MDRAM, the column-hit probability is now 91 percent, and the likelihood of a miss is only 9 percent. That means, the peak bandwidth is only burdened by full row-access latencies  9 percent of the time. If the SDRAM and MDRAM had identical peak bandwidths, the MDRAM's net bandwidth would be much higher than the SDRAM's. Put another way, a MDRAM with a much lower clock rate would produce comparable net bandwidth to that of a much-higher-clock-rate SDRAM.

Figure 2. MoSys DRAM Block Diagram

## Bandwidth That Leads System Requirements

The latency problem is the prime shortcoming of other DRAM alternatives, and by solving it, the MDRAM has a huge leg up on competitive memories. But net bandwidth is a proportion of peak bandwidth, and with system demand growing insatiably, the MDRAM has to address the peak bandwidth issue, too. And it does.

As one would expect, those 32KByte memory banks, built with 16 Mbit memory technology, are very small and very fast. At a clock rate of up to 166 MHz, the bank peak bandwidth is 664 MBytes per second. The internal bus and simple interface repeater provide the internal/external memory bus interface and do nothing to degrade that high bank bandwidth. Thus, the architecture delivers high bank bandwidth right to the package's pins. This bandwidth puts the MDRAM firmly at the front of the memory performance curve, well ahead of other existing memories, and ahead of the spiralling demand for system memory bandwidth.

Finer Granularity

Figure 3. Net Bankdwidth of Various DRAMs

Today, “64-bit DRAM” graphic controllers have become mainstream. But a 64-bit graphics controller with only 1 MByte of DRAM will deliver only half the performance of the same controller fitted with 2 MByte.  This is because the granularity of standard DRAM is such that 1 MByte provides 32 bits and 2 MBytes provides 64 bits.  What about 64 bit performance in 1 MByte? With MDRAM, designers can configure a 1 MByte memory solution with no compromise in performance – 1 giga-byte of peak bandwidth!

A more demanding controller capable of supporting contemporary multimedia applications supporting 1024x768 True color minimally needs 2.25 MBytes of memory. But, here again, designers have to buy much more memory than they need. A 4 MByte memory sub-system to get that 2.25 MBytes of capacity. With its finer granularity, MDRAMs can provide exactly 2.25 MByte capacity – or alternatively 2.5 MByte or 3 MByte if offscreen memory is required.

MDRAM's multibank architecture enables this unusually fine granularity.  Since the banks are fully independent, the total memory capacity is merely the sum of all of the banks.  As a result, MoSys can accommodate designers with devices that are far better tailored to specific design requirements.

## High Memory Bus Efficiency

As previously described, low memory bus efficiency results in more complex and costly design-in. Conversely, high memory bus efficiency results in simpler, less costly design-in. Because it solves the latency problem, the MDRAM also enjoys a higher memory bus efficiency (e.g. net bandwidth/peak bandwidth). For a given bandwidth requirement, designers can use the MDRAM's less complicated interface and fewer pins, yet clock the chip at a lower rate than would be required of alternative DRAM solutions.

## No Cost Problem

Unlike the new "old" DRAMs, the MDRAM does not add a new interface on the classical DRAM architecture, it changes the DRAM architecture. In the process, little or no new chip area is required. The banks are extremely compact for reduced die size. With elimination of extraneous functions, modes, and special features, plus optimally wide internal data paths, the bank becomes a very efficient unit of memory. High bank yields are ensured through traditional row and column design redundancy.

Common function areas of the chip are kept to a minimum. The banks occupy approximately 96 percent of the die. The remaining 4 percent is divided up into common functions: the internal bus, the repeater, and the pads. Minimizing this area is beneficial. It is overhead that does not add function to the memory, and a disabling defect in this area renders the die useless. By keeping the common area small, the overhead is kept small and so is the probability for disabling defects in this area.

The banks also provide a third level of redundancy making them a significant yield enhancement technology. Because they are independent, they decode their own address. Their address may be reprogrammed allowing a spare bank to replace a defective one with no loss in performance.

Together, these MDRAM attributes ensure that die size and manufacturing costs remain comparable to that of standard DRAM, and lower than DRAMS that employ conventional, evolutionary or revolutionary modifications to classic DRAM interface.

## The Overall MoSys MDRAM Advantage

The variety of new DRAM solutions chasing new memory sockets is testimony to the gap between graphics system bandwidth need and DRAM bandwidth performance. The MoSys MDRAM architecture not only closes that gap, it exceeds those requirements. This offers innovative graphics accelerator designers an opportunity to build their new designs based on MDRAM and leave competitive solutions in their vapor trails.

MoSys' vision was to build a DRAM that matches the way that today's systems want to use them. The MDRAM's Multibank architecture is organized the way that systems use memory. Its high peak and net bandwidth capabilities more than satisfy current and near-term bandwidth demand, and the architecture has lots of room for increased peak bandwidth. The MDRAM interface is uncomplicated and easy to integrate. And its streamlined features and functions keep its die size and manufacturing cost commensurate with that of standard DRAM products.

## MoSys Incorporated

## Technology White Paper

## November 1995