MAY  1994

MS Windows for Workgroups Installation With UMC Cards

========================================================

This driver diskette includes an OEMSETUP.INF file for Windows for Workgroups.

Using the OEMSETUP.INF file, the user can select whichever UMC network

adapters are to be installed based on Windows dialog menus.

The OEMSETUP.INF file contains selections for UMC UM9008 network adapter.

Installation procedures:

1. Proceed to install Windows for Workgroups.  If you already have an installed

Windows for Workgroups, open the Control Panel Window to change the network

settings.

2. When Windows asks for the type of network adapter, it will show a list of

all network adapters supported by Windows.  You must select 'Unlisted or

Updated Network Adapter'.

3. Windows will ask for an OEMSETUP.INF file. Since the UMC OEMSETUP.INF

file is included in the UMC driver diskette, insert the driver diskette

in drive A:

Input the full path name where the UMC OEMSETUP.INF file is located.

(i.e. A:\NDIS2\OEMSETUP.INF)

4. You will see a Windows dialog menu listing all UMC network adapters

included in the OEMSETUP.INF file.  Select the network adapter that you will

be using.

5. After selecting the type of UMC network adapter, Windows will look for

the NDIS driver of the network adapter you have selected.

At this time, the UMC driver diskette should still be in drive A:.

Input the full path name where the UMC NDIS driver is located.

(i.e. A:\NDIS2\DOS\)

6. Windows will copy the NDIS driver file and update the Windows network system

automatically.  Be sure to configure the network adapter if you are using

I/O port or IRQ other than the default.

At this point, you can finish the installation or continue the process

according to your preferences.

UMC NDIS PROTOCOL.INI User Guide

===================================

This guide lists the available PROTOCOL.INI keywords for all UMC NDIS device

drivers.  Most of the keywords are optional and have default values.  The

asterisk symbol (*) indicates the list of UMC adapters where the keyword is

applicable.

IOBASE    - This keyword specifies the base I/O port address used

by the UMC Network Interface Card.

INTERRUPT - This keyword specifies the interrupt line used by the

UMC NIC.

DRIVERNAME- This keyword specifies the NIC driver's name.  The NIC

driver name follows the NIC adapter type. (i.e. UM9008)

ADAPTERID - This keyword specifies the node address (hex) used by

your network interface card.  This option is used only

if you want to override the node address burned in your

NIC.  In this case, the specified address will be used

instead of the NIC's actual node address.