Server Driver for NetWare 3.11

Introduction:

-------------

This document describes the procedure to install the NetWare v3.11

server driver for Genius LAN GE2500III SE PCI Ethernet adapter.

Location of Driver:     \NETWARE\NW311\PCISRV.LAN

Installation Procedure:

-----------------------

Before you start with the installation  process,  make sure that

the Novell NetWare 386 v3.11 server is properly installed.

Similarly, your adapter should also be properly installed in your

workstation.

1. If you can log into the server as a supervisor, copy PCISRV.LAN

relative  files from the Genius LAN GE2500III SE Diskette  subdirectory

A:\NETWARE\NW311 into the NetWare 386 subdirectory SYSTEM of your

server. (If some files exists, rename the  existing files in the

SYSTEM subdirectory.)

2. Run  SERVER.EXE  from  your  server.  You will see a series of

NetWare messages on screen.

3. When the NetWare 386 prompt appears (indicated by a colon),load

your server driver. Simply type:

: LOAD LSLENH <Enter>

: LOAD PCISRV <Enter>

or, you can directly load file from Genius GE2500III SE driver disk:

: LOAD A:\NETWARE\NW311\LSLENH <Enter>

: LOAD A:\NWSERVER\NW311\PCISRV <Enter>

4. Bind IPX to the adapter driver. At the NetWare server prompt

,type:

:BIND IPX TO PCISRV <Enter>

5. After pressing <Enter> the computer prompts you for the Network

Number. For details on how to assign this number,please consult

your NetWare 386  Installation Manual.

6. Add the load and bind statements you require to the server's

AUTOEXEC.NCF file so that the LAN driver will load automatically

each time the server starts up.

NOTES:

------

1. Installing Multiple LAN Adapters:

The keyword "EtherID" is the ethernet address of a adapter. This

keyword is provided for multiple LAN adapters in a single server

by the driver PCISRV.LAN.  So, add EtherID in LOAD commands. For

example:

LOAD PCISRV EtherID=0123456789AB FRAME=Ethernet_802.2 NAME=LAN_A

BIND IPX TO LAN_A NET=11

LOAD PCISRV EtherID=1234567890AB FRAME=Ethernet_802.3 NAME=LAN_B

BIND IPX TO LAN_B NET=22

2. Installing One LAN Adapter with Multiple Frame Types:

When  binding multiple frame types to one adapter, enter a LOAD

and BIND statement for each frame type. Each LOAD statement will

use a different network number is required on the bind statement.

You need  to supply  a name on each load line in order to avoid

being  prompted  for which  board to bind IPX to. If you do not

have the name option in the AUTOEXEC.NCF , it will not  execute

completely without user intervention.  Example:

LOAD PCISRV FRAME=ETHERNET_802.3 NAME=IEE8023

BIND IPX TO IEE8023 NET=77777

LOAD PCISRV FRAME=ETHERNET_802.2 NAME=IEE8022

BIND IPX TO IEE8022 NET=88888

If problems occur during the binding process, your  screen will prompt

you with error  messages.  Please refer to your NetWare 386 manual for

details on these messages.