******************************************************************************

*  Genius LAN GE2500III SE PCI Ethernet Adapter  -- NetWare Driver                *

******************************************************************************

Driver installation Guide for NetWare OS2 ODI client

==========================================================

Introduction

============

This document describes the procedure to install Netware OS2 ODI

driver for Genius LAN GE2500III SE PCI Ethernet Adapter.

Installation Procedures

=======================

Before you start with the installation procedure, make sure that the

adapter is properly installed and configured.  Similarly, Make sure

that your NetWare Requester for OS/2 is properly installed.

Please refer to "Novell NetWare Requester for OS/2" documentation for

a detailed  Requester and Utilities installation guide.

1. Copy the  PCIOS2.SYS  file into the NETWARE subdirectory of your

hard disk from the Driver Diskette A:\NETWARE\OS2ODI.

2. Add or replace the PCIOS2.SYS in the CONFIG.SYS file.

3. Edit the NET.CFG file that you copied over. Edit the file per your

requirements (see examples above).  If there are two frame types

listed, whichever frame type is listed first, that is the one that

the driver will load.  See above for examples.

4. Reboot your system to get service of network.

--- Sample  CONFIG.SYS file ---

:

DEVICE=C:\NETWARE\LSL.SYS          ; OS/2 Requester supported

DEVICE=C:\NETWARE\DDAEMON.SYS      ; OS/2 Requester supported

DEVICE=C:\NETWARE\PCIOS2.SYS       ; Manufacturer supported

DEVICE=C:\NETWARE\IPX.SYS          ; OS/2 Requester supported

:

--- Sample NET.CFG file ---

:

LINK DRIVER PCIOS2

FRAME ETHERNET_802.3

FRAME ETHERNET_802.2

======================================================================