LANtastic 6.0

Introduction:

-------------

This document describes the procedure to setup NDIS driver on

LANtastic 6.0 operation system for Genius GE2500III SE PCI ethernet

card.

Location of Driver:     \ARTISOFT\6.X\PCIND.DOS

-------------------

Sample Configuration Files:

--------------------------

CONFIG.SYS:

DEVICE=C:\LANTASTI\PROTMAN.DOS /I:C:\LANTASTI

DEVICE=C:\LANTASTI\PCIND.DOS

FILES=40

BUFFERS=30

STARTNET.BAT:

@echo off

rem LANtastic  Version 6.00  installed 95/05/23 14:11:21

rem (for Windows)

C:

cd C:\LANTASTI

SET LAN_CFG=C:\LANTASTI

rem If LANtastic is disabled, skip everything.

IF EXIST DISABLED GOTO :STARTNET_DONE

@echo ===== Begin LANtastic configuration =====

PATH C:\LANTASTI;C:\LANTASTI\NW;%PATH%

SET LAN_DIR=C:\LANTASTI.NET

SET NWDBPATH=C:\LANTASTI\NW

rem Please obtain the NETBEUI.EXE program from Microsoft

LOADHIGH NETBEUI

LOADHIGH AI-NDIS BIND_TO=PCIND_NIF

AILANBIO @STARTNET.CFG

REDIR TEST @STARTNET.CFG

IF EXIST NOSHARE GOTO :NOSHARE

SERVER C:\LANTASTI.NET @STARTNET.CFG

NET LOGIN \\TEST

GOTO :CONTINUE

:NOSHARE

@echo LANtastic server was installed but turned off.

:CONTINUE

rem If CONNECT.BAT exists, run it to set up connections.

IF EXIST CONNECT.BAT GOTO :CONNECT

rem Otherwise set up connections specified during install.

NET LPT TIMEOUT 10

GOTO :CONNECT_DONE

:CONNECT

@echo Setting up LANtastic connections from CONNECT.BAT

rem Build CONNECT.BAT like this: "NET SHOW/BATCH >

C:\LANTASTI\CONNECT.BAT"

rem   (or run the batch file SETNET.BAT)

call CONNECT.BAT

:CONNECT_DONE

NET POSTBOX

@echo ===== End LANtastic configuration =====

:STARTNET_DONE

cd \

PROTOCOL.INI:

;PROTOCOL.INI for LANtastic Version 6.00

;------------ using

;

[PROTMAN]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

[PCIND_NIF]

DRIVERNAME = PCIND$

Adapters=RTL8029

AdapterID=@0123456789AB

Install Genius drivers for LANtastic 6.0 : (Using NDIS driver)

------------------------------------------

The  installation  procedure  will  transfer  files  to a  specific

directory on the workstation and modify existing configuration files

to fit your specific needs.  The installation utility INSTALL.EXE is

located on LANtastic Network Software Disk 1.   Run INSTALL.EXE from

Disk 1  and  the  LANtastic  Install program screen will appear. You

must  complete each of the questions.  These options are specific to

your needs.

1. Prompt you to enter a unique Computer Name.

2. Ask you to  select  the drive that you want LANtastic to

be installed on. The default is Drive C:.

3. Ask  you  to  specify  a  directory  where  you want the

LANtastic files to be installed. The default is \LANTASTI.

4. Ask  you  if you want to Share your Computers Drives  or

Printers.

5. Ask  you  to  select  the  Maximum  Number  of Connected

Computers.

6. Prompt  you  to  select any other LANtastic features that

you may want to install.

7. A summary  screen of the  options you will appear. If it

looks  correct  continue  with  the  installation.

8. Prompt  you  to select a  network  adapter  to  run your

LANtastic software on. From the list of adapters , select

"NDIS SUPPORT FOR NETWORK ADAPTERS".

9. Ask  you to insert the  manufacturers   driver  diskette

containing the driver files. Insert the  RTL8029  driver

diskette into drive A:  and  specify the source path for

driver files as A:\

10. Ask you if you want to Set Up Permanent Drive or Printer

Connections.

11. Another summary screen of the  options  will appear.  If

this looks correct then select Perform the Installation

and LANtastic will now install the files.