Packet Driver

Introduction:

-------------

This document describes the procedure to setup the Packet driver for

Genius LAN GE2500III SE PCI Ethernet card.

Location of Driver:     \PKTDRV\PCIPKT.COM

Sample Configuration Files:

---------------------------

AUTOEXEC.BAT

PCIPKT 0x60

Notes:

------

1.  Load the packet driver using the software interrupt and any

optional switches if required.

Usage: PCIPKT [options] <software_int_no>

Example: PCIPKT -i 0x7e

The only required parameter is the software interrupt. A packet

driver needs to have a software interrupt assigned to it for other

programs to access it.  The recommended interrupt 0x7e.  If you need

any of the options listed below, place them before the software

interrupt on the command line.

-i - Force driver to report itself as 802.3 instead of Ethernet II.

-d - Delayed initialization.  Used for diskless booting

-n - NetWare conversion.  Converts 802.3 packets into 8137 packets

-w - Windows hack, obsoleted by winpkt

-p - Promiscuous mode disable

-h - Help

-u - Uninstall

3. Type command,C:\PCIPKT 0x60, the screen will appear following

message:

System: [345]86 processor, PCI bus, Two 8259s

Packet driver software interrupt is 0x60

Interrupt number is 0x9

I/O port is 0x6100

My Ethernet address is 52:54:4C:29:29:AD

4. Installing Multiple LAN Adapters:

If there are multiple network cards on the main board. the user

run PCIPKT 0x60 directly, then the screen will display :

Packet driver for RTL8029, PCI version 1.00

Copyright 1995 (c), Realtek Semiconductor Inc.

There are 2 network cards on your main board :

Card 1. IRQ= 0x9  I/O= 0x6100  NodeID= 52:54:4C:29:29:AD

Card 2. IRQ= 0xA  I/O= 0x6000  NodeID= 52:54:4C:29:29:64

Please select a Card No.(1-2): 1

System: [345]86 processor, PCI bus, Two 8259s

Packet driver software interrupt is 0x60

Interrupt number is 0x9

I/O port is 0x6100

My Ethernet address is 52:54:4C:29:29:AD