Driver Installation Guide for SCO UNIX 4.x

================================================

Introduction

============

This document describes the procedures to install the SCO UNIX driver

for PCI Slave Ethernet Adapter.

The following files will be used in this installation:

Files          Contents

-----------    -----------------------------------------

driver.o       SCOPCI Driver Program

setup          Transfer dos format to unix format setup program

info           Used by netconfig

init           Used by netconfig

master         Used by netconfig

node           Used by netconfig

reconf         Used by netconfig

remove         Used by netconfig

space.c        Used by netconfig

system         Used by netconfig

Installation Procedures

=======================

Before you start with the installation  process,  make sure that the

SCO UNIX system is properly installed. Similarly,your adapter should

also be properly installed in your machine.

1. Login as root user. (On maintenance mode)

2. (a) If you own installation Diskette in UNIX format :

# custom

Select "Install".

Select "a New Product".

Select "entire" to install all of the packages.

(b) If you own installation Diskette in DOS format :

Put SCO UNIX Driver diskette to floppy A: and use doscp

command to copy the file "setup" for installing SCO UNIX

PCI Driver into UNIX's directories as following :

# cd /

# doscp A:/sco/setup /setup

!! if you don't have the directory ,namely, "a:/sco/4.x"

in the disk, you should modify contents of the setup file in

the floppy. Please check contents of the setup file.

# chmod +x /setup

# ./setup

3. Use netconfig utility

Avaiable options :

# netconfig

The screen will display the following messages, and Select

item (1) to Add a chain  :

1) Add a chain

2) Remove a chain

3) Reconfigure an element in a chain

4) Quit

4. After the above option completed, the screen display next option

messages :

Select item (4) : sco_tcp   SCO TCP/IP for UNIX

5. When the user completed  the  above option, then various adapter

drivers in SCO UNIX occur and select " GE2500III Ethernet

Driver board 0" item.

6. Add chain sco_tcp -> pci0 (y/n)                     TYPE 'y'

7. Enter the internet Address of this interface.

for example : 192.9.100.1

8. Enter the netmask for this interface (default 255.255.255.0).

9. Does the interface use a broadcast address of all 1's (y/n).

default : y

10. Enter the boardcast address for this interface.

default : xxx.xxx.xxx.255                           press ENTER

11. Are these values correct ? (y/n)                    TYPE 'y'

12. Pseudo ttys are currently configured, do you want to:

1) Add Pseudo ttys

2) Remove Pseudo ttys

Select an option or enter q to quit [q]          TYPE 'q'

13. Available options:

1) Add a chain

2) Remove a chain

3) Reconfigure a element in a chain

q. Quit

Select option: q

14. Do you want to relink to kernel now ?               TYPE 'y'

15. Do you want this kernel to boot by default (y/n)    TYPE 'y'

16. Do you want the kernel environment rebuilt (y/n)    TYPE 'y'

17. Reboot unix, type following command.

# shutdown -y -g0