******************************************************************************

*        NDIS Driver Installation Guide for Banyan VINES Client              *

******************************************************************************

Introduction

============

This document describes the procedure to install Banyan VINES DOS

client driver (NDIS 2.01) for Genius LAN GE2500III SE PCI Ethernet

Adapter.

These drivers conform to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedures

=======================

1.  Use the DOS MD command to create a subdirectory called \VINES.

2.  Copy the contents of the VINES LAN S/W (DOS) Install diskette

into the \VINES subdirectory.

3.  Copy the contents of the NDIS subdirectory on the VINES LAN S/W

(DOS) 2 of 2 diskette into the \VINES subdirectory.

4.  Copy all files from \VINES subdirectory on the driver diskette

to the \VINES subdirectory.

5.  Edit the PROTOCOL.INI file to read as follows:

[PROTOCOL MANAGER]

DRIVERNAME = PROTMAN$

[VINES_XIF]

DRIVERNAME = NDISBAN$

BINDINGS = RTL8029_NIF

[RTL8029_NIF]

DRIVERNAME = PCIND$

6.  Run the PCCONFIG.EXE program located in the \VINES subdirectory.

7.  Select option 1, Network Card Settings.

8.  Select the "NDIS Ethernet" option.

9.  The NDIS workstation screen appears. Enter the IRQ that you

selected for the PCI Slave Ethernet adapter.

10.  Press 'F10' when done, press <ESC> to exit this screen.

11.  Select "2 - Login Environment Settings. ", Press <Enter>.

12.  Select "1 - Select Default Communications Driver. ", Press <Enter>.

13.  Choose "NDIS Ethernet", Press <ESC> to exit this menu.

14.  Edit the CONFIG.SYS file and add the following lines, substituting

A: for C: for a floppy diskette installation.

DEVICE = C:\VINES\PROTMAN.DOS /I:C:\VINES

DEVICE = C:\VINES\PCIND.DOS

15.  Add the BAN command to the AUTOEXEC.BAT file.  BAN loads the NDISBAN

drivers and reads the PCCONFIG.DB file to invoke changes made during

the PCCONFIG stage.