******************************************************************************

*  Genius LAN GE2500III SE Ethernet Card Windows for Workgroups 3.1 Driver   *

******************************************************************************

Introduction

============

This document describes the procedure to install Microsoft Windows

for Workgroups v3.1 driver for GE2500III SE PCI Ethernet Adapter.

These drivers conform to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedures

=======================

A. Install driver when WFW 3.1 doesn't exist.

1. When installing Windows for Workgroups, you are prompted for a

network driver. Select the "Unlisted or Updated Network Driver".

2. Insert the adapter driver disk.

3. When a dialog screen is displayed on the screen to ask the driver

path. Enter "A:\WFW31" or "B:\WFW31" and press return or OK button.

4. Select Advanced options, and make sure the parameters match the

card settings. The installation program copies the necessary files

and creates PROTOCOL.INI for use with the adapter.

5. Continue the installation per Windows for Workgroups instructions.

B. Install driver when WFW 3.1 already exists.

1. Start your Windows For Workgroups and open main group.

2. Select "CONTROL PANEL" icon in main group.

3. Select "NETWORK" icon in control panel window.

4. Select "ADAPTERS" icon in network window.

5. Select the Add option.

6. Select "Unlisted or Updated Network Adapter" and press OK button.

7. Insert the adapter driver disk.

8. When a dialog screen is displayed on the screen to ask the driver

path. Enter "A:\WFW31" or "B:\WFW31" and press return or OK button.

9. After Windows has read the needed files. It will show another dialog

screen. You can select hardware configuration as your adapter settings.

10. After Windows stores the setting, You can restart your computer to

start the driver.

C. TO ADD NETWARE.

1. Select the "Control Panel" icon in the Main Group.

2. Select the "Network" icon.

3. Select the "Networks" button.

4. Select "Novell Netware" and select the add button.

5. Select "OK" and close the Network icon.

6. Select reboot computer and restart Windows.