. This diskette contains the newest LANSET program Ver 3.05 [Date:1994.11.24]

. What is LANSET.EXE ?

. A setup program for jumperless LAN cards in a very easy operation

. Integrated setup for multiple kinds of LAN cards and with auto-

detecting when setup startup, it also implements onboard diagnostic

. Supports up to 4 LAN cards may con-currently installed in a PC system

(as for bridge/router multi-LAN-cards systems, testings, and so on).

. Supports to try to rescue from some common I/O port collision cases

without remove any hardware adapters.

. What kinds of LAN cards are supported ?

. Setup 16-bit jumperless LAN cards:

LAN2Nx  - LAN Chipset: NS DP83906x

LAN2Ux  - LAN Chipset: UMC 9003x

LAN2Mx  - LAN Chipset: MYSON's MTD900F EE/EF, MTD908x

. View setting only for jumper LAN cards:

LAN2x   - LAN Chipset: DP8390x / other NE2000 compatible cards

. About 10-Base series: (Line-interface)        (Line-distance for maximun)

. 10-BaseT      Twisted-pair UTP        100 Meter

. 10-Base2      BNC / Thin-Coaxial      200 Meter

. 10-Base5      AUI 9-pins transfer     500 Meter

(AUI needs external transfer connect to Thick Ethernet cable)

. Histories (Backward)

[v3.05] . Fixed: LAN2Mx Diagnostic's loopback test skipped level 2/3

[v3.04] . Fixed: Diagnostic test fails when line is busy

[v3.03] . Improve rescue process of LAN2Ux/LAN2Nx: auto fix EEPROM

data (16/8-bit mark) for it maybe due to auto-detect error

. Add /RESCUE command-line option to force start rescue menu

[v3.02] . Fixed: Un-selectable for each third item of setup menu item

. Mark un/selectable item of Rescue menu's sub-menu:

LAN2MX-HIPORT I/O ports {for Chipset MTD900F EE only}

[v3.01] . More support rescue LAN2Mx for chipset MTD900F EE only

[v3.00] . Support auto boot-up rescue-menu select when startup

not found any LAN card for rescue from I/O collision cases

[v2.95] . Enhance Onboard DMA RAM buffer testing (upto 6 patterns)

[v2.94] . Enhance auto-detect procedure for LAN2Nx/LAN2Ux

. Remove LAN-2Nx media-select (hardware/UTP auto-detectable)

[v2.93] . Enhanced LAN-2Nx {NS83906} stablity (fix some EEPROM data)

[v2.92] . Add a LAN-2Ux cards Diagnostic item: Onborad Buffer RAM test

[v2.91] . Support LAN-2Nx & other NE2000 Ethernet compatible cards'

diagnostic for onboard DMA RAM buffer testing,

slot-bus 8/16 bits number identifing

[v2.90] . Support LAN-2Nx cards (LAN chipset: NS DP83906)

. Add switches: /N /N=port (see LANSET /? for detail)

. Fixed v2.82 LAN-2Ux's IRQ select "IRQ 14" --> "IRQ 12"

. Screen hardware cursor make more smooth than before

[v2.82] ( Bugs fixed since last released version v2.81 )

. For LAN-2Mx card (LAN-Chipset: MYSON's MTD900F vEE or vEF),

Make effective for NE2000/NE1000 mode selection (advanced)

. Fixed LAN-2Ux sometime rejects to enter the main setup menu

(It maybe can skips by using /UFO switch for last version)

[v2.81] . In order to make more stable for LAN chipset: MTD900 EF,

default set Polarity to Off and Disable Media's Auto-Switch

[v2.80] . Support MTD908 chipset (one of LAN-2Mx cards)

. Add switches: /M=8

[v2.7]  . Completed supported for both UMC9003x and MTD900Fx only

. Completed setup-data load/save for above both cards

. Support Mono-VGA colors

. Add switches: /U /U=port /M /M=port /M=E /M=F

[v2.6]  . Tested usable for both UMC9003x and MTD900Fx

[v2.5a] . Support menu color / underline in color / mono display

. Splits common ETHERNET I/O menu and LANChipset advanced menu

. Remove Link-Test option

[v2.12] . Support command line option to set MYSON chipset version /VE?

[v2.1]  . More support a new chipset: MYSON900F EF (old was EE)

in only one version !!

. Turn off BNC DC-converter power when use UTP (twisted-pair)

(For more power saving)

[v2.01] . Let Onboard RAM buffer size checking result more stable

(Durring RAM checking, it also receive broadcast packet !!

this is the common problem for most LAN card setup programs)

[v2.0]  . Enhanced internal card reset logic for faster & more stable

. Internal IRQ-testing add EOI before iret (interrupt return)

[v1.9]  . Fixed problem: Set IRQ 2/9 will fail (v1.4 & before is OK)

. Change internal I/O timming rate double (*2) to triple (*3)

(For some critical machines)

. Support CPU type detecting

(Identifies for IRQ 2 or 9, if enable IRQ 9 ~ IRQ 15)

[v1.8]  . Enhance link test: stablity, Tx/Rx OK result display

[v1.7]  . Add more detail internal setting

[v1.6]  . More user message, smaller main-menu display size

[v1.5]  . More testing/updated for Multi-Cards link test

[v1.4]  . Add Multi-card transmitting/receive test

[v1.3]  . Add transmitting/receive test

[v1.2]  . Fixed interrupt share (ISH) to 0

. Support /B: Enable to set Boot ROM address under EMS driver

. For more stablity, double timing

. Restore physical addr when loopback test

[v1.0]  . First released

. Append for about LANSET command lines options

. It is normally NO need any command line options for most cases

. It can display belows text by only enter the "LANSET /?" command

-------------------------------------------------------------------------------

<16-bit jumperless> LANcard SETup v3.03  NE2000 ETHERNET Compatible [2N/2U/2M]

Usage>  LANSET  [/Options see below ...]

Common: /RESCUE Force startup RESCUE menu {for I/O collision} (Default:Auto)

/VM     Specifies to use colors for Mono-VGA video interface

/T=rate Specifies I/O timming rate on un-stable machines (Default /T=4)

/I      Do NOT mask IRQ interrupts. (Default mask card's IRQ in setup)

LAN2Nx: /N      Only allow to setup LAN2Nx {chipset: NS DP83906x} cards

/N=port Find & Set a LAN2Nx card to the specified I/O port

LAN2Ux: /U      Only allow to setup LAN2Ux {chipset: UMC9003x} cards

/U=port Find & Set a LAN2Ux card to the specified I/O port

LAN2Mx: /M      Only allow to setup LAN2Mx {chipset: MTD90x} cards

/M=port Find & Set a LAN2Mx card to the specified I/O port

/M=E    Assign all LAN2Mx cards' chipset to MTD900F version EE

/M=F    Assign all LAN2Mx cards' chipset to MTD900F version EF

/M=8    Assign all LAN2Mx cards' chipset to MTD908

/M=C    Assign all LAN2Mx/LAN2MC cards' connect media to BNC/Coaxial

/M=T    Assign all LAN2Mx/LAN2MT cards' connect media to UTP/Twisted

Note:   port:           A heximal number for I/O port address

Find & Set:     This may helps to rescue from some collision cases,

Especially for those from non-operative ports.

After this option, it needs to power-off and restart!

-------------------------------------------------------------------------------

. To setup adapter cards , a set of parameter value is recommended.

STANDARD SETUP PARAMETERS

-------------------------

IO base address         300,320,340,360 or

(MYSON Series)          200,220,240,260

(UMC Series : *)        300,320,340,360 or

240,280,2C0

IRQ number              3,4,5,9,10,11,12,15

Node Id                 Set Ethernet node ID

ADVANCED SETUP PARAMETERS

-------------------------

Boot ROM address        C000 to DC00 or Disable

Cable type              BNC(AUI), RJ-45 or Auto Detect

Threshold               Adjust threshold, from 125mV to 370mV

Polarity                Enable or disable polarity correction

*  CHRDY Control           From IO R/W or BALE

*  IO16 Control            Address Decode  or After IO R/W

*  Fast Read               Disable or Enable

RECOMMENDED VALUES FOR SETUP PARAMETERS

----------------------------------------

Threshold               370mV

Polarity                Enable

IO base address         300H

IRQ number              3

Boot ROM address        C800H

Media type              Auto Detect

CHRDY Control           From IO R/W

IO16 Control            Address Decode

Fast Read               Disable

//