MircoSoft Windows for Workgroups Installation

With Lan-2x adapters series

=============================================

=========================

This driver diskette includes an OEMSETUP.INF file for Windows for Workgroups.

Using the OEMSETUP.INF file, the user can select whichever LAN-2x network

adapters are to be installed based on Windows dialog box menus.

The OEMSETUP.INF file contains selections for the following LAN-2x series

network adapters:

Lan1 = "Lan-1x (NOVELL's NE1000 Compatible) adapter"

Lan2 = "Lan-2x (NOVELL's NE2000 Compatible) adapter"

LanWD1 = "Lan-3x (WD8003 Compatible) adapter"

LanWD2 = "Lan-4x (WD8013 Compatible) adapter"

Installation procedures:

1. Proceed to install Windows for Workgroups.  If you already have an

installed Windows for Workgroups, open the Control Panel Window to

change the network settings.

2. When Windows asks for the type of network adapter, It will show a

list of all network adapters supported by Windows.  You must select

'Unlisted or Updated Network Adapter'.

3. Windows will ask for an OEMSETUP.INF file. Since the LAN-2x OEMSETUP.INF

file is included in the LAN-2x driver diskette, insert the driver diskette

in drive A:

Input the full path name where the SuperLan OEMSETUP.INF file is located.

(i.e. A:\NDIS\OEMSETUP.INF)

4. You will see a Windows dialog menu listing the LAN-2x network adapters

included in the OEMSETUP.INF file.  Select the network adapter that

you will be using.

5. After selecting the type of LAN-2X network adapter, Windows will look

for the NDIS driver of the network adapter you have selected.

At this time, the LAN-2x driver diskette should still be in drive A:.

Input the full path name where the LAN-2x NDIS driver is located.

(i.e. A:\NDIS\)

6. Windows will copy the NDIS driver file and update the Windows network

system automatically.  Be sure to configure the network adapter if you

are using I/O port or IRQ other than the default.

At this point, you can finish the installation or continue the process

according to your preferences.

7. After installation is completely finished, restart your system for the

updates to take effect.

#  Notation :

# The interrupt selected can not conflict with other interface card on

your computer.

# The I/O base selected can not conflict with other interface card on

your computer

# You could refer to your menu to select interrupt and I/O base by

adjusting switches.