### EEPROM Layout for VT6102 and Integrated LAN of VT8231/VT8233/VT8235/VT8237

b15							b8	b7							b0

| ETHER_ID1 | ETHER_ID0 | 00h |
| --- | --- | --- |
| ETHER_ID3 | ETHER_ID2 | 01h |
| ETHER_ID5 | ETHER_ID4 | 02h |
| Reserved | MII_PHY_AD | 03h |
| SUB_SID1 | SUB_SID0 | 04h |
| SUB_VID1 | SUB_VID0 | 05h |
| Reserved | Reserved | 06h |
| Reserved | Reserved | 07h |
| Data_SEL | PMCC | 08h |
| AuxCurr | PMU_DATA_REG | 09h |
| Reserved | Reserved | 0Ah |
| Max_LAT | Min_GNT | 0Bh |
| BCR1 | BCR0 | 0Ch |
| CFG_B | CFG_A | 0Dh |
| CFG_D | CFG_C | 0Eh |
| CHKSUM | 73h | 0Fh |

### 2. EEPROM Content

| Word Offset | Bit | Default Value by EEPROM Utility | Name | Description |
| --- | --- | --- | --- | --- |
| 00h ~02h | [47:0] | 000000000000h | ETHER_ID | Ethernet address. Default is an invalid value. Customers SHOULD replace it with a valid Ethernet address. |
| 03h | [7:0] | 00h | MII_PHY_AD | PHY address. Default is an invalid value. Customers SHOULD replace it with a correct PHY address according to PCB layout. |
|  | [15:8] | 00h | Reserved | Reserved. Recommend using the default value. |
| 04h | [15:0] | 0102h | SUB_SID | Subsystem ID. Default is an invalid value. Customers SHOULD replace it with their own Subsystem ID. Subsystem IDs are vendor specific. |
| 05h | [15:0] | 1106h | SUB_VID | Subsystem Vendor ID. Default is an invalid value. Customers SHOULD replace it with their own Subsystem Vendor ID. Subsystem Vendor IDs can be obtained from the PCI SIG. |
| 06h | [15:0] | 3065h | Reserved | Reserved. Recommend using the default value. |
| 07h | [15:0] | 1106h | Reserved | Reserved. Recommend using the default value. |
| 08h | [7:0] | 1Fh | PMCC | Power Management Capabilities Control Register. |
|  | [15:8] | 10h | Data_SEL | PM data select register. |
| 09h | [7:0] | 00h | PMU_DATA_REG | PM data register. |
|  | [15:8] | 00h | AuxCurr | 3.3Vaux auxiliary current. |
| 0Ah | [15:0] | 0000h | Reserved | Reserved. Recommend using the default value. |
| 0Bh | [7:0] | 03h | Min_GNT | Minimum grant. |
|  | [15:8] | 08h | Max_LAT | Maximum latency. |
| 0Ch | [7:0] | 09h | BCR0 | Bus Control Register 0. |
|  | [15:8] | 0Eh | BCR1 | Bus Control Register 1. |
| 0Dh | [7:0] | 03h | CFG_A | Configuration register A. |
|  | [15:8] | 00h | CFG_B | Configuration register B. |
| 0Eh | [7:0] | 40h | CFG_C | Configuration register C. |
|  | [15:8] | 82h | CFG_D | Configuration register D. |
| 0Fh | [7:0] | 73h | PROG_STATUS | EEPROM programmed status. The value 73h means this EEPROM had been programmed. |
|  | [15:8] | 73h | CHKSUM | EEPROM checksum. Default value is 73h. The checksum is not used at current stage, any value here will be OK. |

- BootROM size

CFG_C: Word Offset 0Eh[7:0] (for VT6102 only)

| Bit | Default Value | Name | Description |
| --- | --- | --- | --- |
| [2:0] | 0,0,0 | BootROM_Size selected | 0,0,0 = No BootROM 0,0,1 = 8K 0,1,0 = 16K 0,1,1 = 32K 1,X,X = 64K/FlashROM 8K,16K,32K are for legacy use. For VT6102, always use 1,X,X if there is a BootROM. |

- Boot option

CFG_A: Word Offset 0Dh[7:0]

| Bit | Default Value | Name | Description |
| --- | --- | --- | --- |
| [4:3] | 0,0 | Boot_Option | When CFG_D[5] = 0: 0,0 = Hook int 0x19 0,1 = Hook int 0x18 1,0 = Boot from Local Disk 1,1 = BEV (Boot Entry Vector) When CFG_D[5] = 1: 0,0 = Boot from Network Server 0,1 = Reserved 1,0 = Boot from Local Disk 1,1 = Reserved |

- PXE/RPL boot select option

CFG_D: Word Offset 0Eh[15:8]

| Bit | Default Value | Name | Description |
| --- | --- | --- | --- |
| [5] | 0 | PXERPL_Option | 0 = PXE boot 1 = RPL boot |

NOTE: Devices can work well by using most of the recommended default value. But the values from word offset 00h~05h should be taken care very much, or the device will NOT function.