PCI56RVP Installation Introduction for Windows NT

.Before Installation.

.Copy the file PCI56NT.EXE to HDD(C:\) from the driver diskette.

.Run c:\PCI56NT.EXE to decompress the NT driver .(c:\PCI56NT\ )

.Installing the PCI56RVP.

- Run C:\PCI56NT\Setup.exe to install PCI56RVP modem.

- Click “NEXT” and make sure the MODEM is correct.

- Click “NEXT” to copy the files to your system.

- Click “Finish” and Restart your PC.

Note: Now, please check “control panel Modem” and find the PCI56RVP Modem.

.Installation Remote Access Service.

- Double click ”Network” of “Control-Panel”.

- Select “Service” and double click “Remote Access Service” and open

‘Remote Access Setup’

- Click ”Add” button to add RAS device .

- Select “COM3 PCI56RVP Modem “. Then click ”OK” button.

- Click “Configure” button.

- Check the radio box” Dial out and Receive calls “ . And click “OK” to countinue.

- Click “continue” to copy the date to your system ,and return to Network-BOX.

- Click “Close” and let your system run ‘Link setting” ……

- Click ”Yes” to Restart PC and Change network setting .

.Setting Dial-Up Networking.

- Double click the “dial-up networking” icon of “My Computer”.

- Please fill out the name of the new Phonebook Entry Wizard. Then click” Next” button.

- Check the box ”I am calling the the Internet” and the box” Send my plain text password

if that’s the only way to connect”. Then click “ Next “ button.

- Please fill out the phone number of your ISP. Then click “Next” button.

- One new phonebook entry has created .Click “Finish” button to save this new Phonebook

Entry.

- Click “More” button. And point to “ Edit entry and Modem Properties” item.

- Make Sure the information is correct in the Basic menu.

- Click “Server” menu .Check the box “TCP/IP”. Then click “TCP/IP Setting” button.

- Fill out your IP address and address of the domain name servers. Get this information

from your ISP. Click “OK” to continue.

10.Click” Security” menu. Only check the radio box “Accept any authentication including

clear text” . Click “OK” button to finish the configuration of phonebook entry.

11.Click “Dial” button.

12.Please Fill out your User name and Password. Leave the Domain field blank. Then

click the “OK” button to connect to your ISP .After the call is connected, you can

use various TCP /IP application like FTP,Gopher,News,e-mail,etc…