FVGAP-SS6 VGA CARD

DOS Utility & Driver Installation

### 1 Unpack & Copy

To unpack and copy drivers to where they should resides, please follow the following procedures:

1. 	Run "INSTDRV.EXE" where it resides.

2. 	In "SiS Super VGA Drivers Installation" menu,

select "A. ADI 4.2" to unpack and copy drivers.

(To select, type "A".)

5. 	In "Unpack & Copy ADI 4.2 Drivers" screen,

keyin the "drive:\directory" where these drivers would reside

(default C:\ADI42).

Program would unpack & copy all related driver files to where you assign.

6. 	After "unpack and copy" completed,

exit the INSTDRV.EXE program.

7. 	Refer to Sec. 4.1 to Sec. 4.4 for the real installation procedures for each program.

2 AutoCAD R11 Setup

1. 	The following procedures assume that

(1) You have complete "unpack & copy" procedure.

(2) Your ADI 4.2 drivers are located in C:\ADI42.

2.	Add the following setting to your own batch file for AutoCAD R11 (say ACADR11.BAT) or to your "AUTOEXEC.BAT" file:

SET DSPADI=\ADI42\RCPSIS.EXP  <Enter>

3.	Delete the configure file ACAD.CFG resides in \ACAD directory.

4.	Type 	ACADR11 	<Enter>  to configure your AutoCAD R11 system.

5.	In "Select Display Device:" item, choose "ADI P386 V4.0/4.1 display.".

6. 	In "Select Display Resolution" screen, choose which display driver you want to use.

7. 	Go through the whole instructions, and the system would start with the desired display 	setting.

3 AutoCAD R12 Setup

1. 	The following procedures assume that

(1) You have complete "unpack & copy" procedure.

(2) Your SiS ADI 4.2 drivers are located in C:\ADI42.

(3) Your AutoCAD R12 program is located in C:\ACADR12.

(4) Your AutoCAD R12 default drivers are located in C:\ACADR12\DRV.

(5) Your AutoCAD R12 configure file ACAD.CFG is located in C:\ACADR12.

2. 	Copy the following driver file to C:\ACADR12\DRV:

RCPSIS.EXP

You may complete this step by

COPY C:\ADI42\RCPSIS.EXP C:\ACADR12\DRV

3. 	Delete your original ACAD.CFG file.

You may complete this step by	DEL C:\ACADR12\ACAD.CFG

4. 	Restart your AutoCAD R12 program as usual.

5. 	AutoCAD R12 will ask you to complete the configuration procedures,since it can't find the

configure file ACAD.CFG.

6. 	Follow the instructions of AutoCAD R12 to proceed configuration.

7. 	In "Available Video Displays:" item, choose the "SiS Super VGA ADI v4.2 Display and Rendering driver" item.

8. 	In "Select Display Resolution" screen, choose which display driver you want to use.

9. 	Go through the whole instructions, and the system would start with the selected display setting.

4 AutoShade R2.0 Setup

1. 	The following procedures assume that

(1) You have complete "unpack & copy" procedure.

(2) Your ADI 4.2 drivers are located in C:\ADI42.

2. 	Add the following settings to your batch file for AutoShade R2.0 (say SHADE2.BAT) or to your "AUTOEXEC.BAT" file.

(a) For display driver setting,

SET DSPADI=\ADI42\RCPSIS.EXP

(b) For rendering driver setting,

SET RDPADI=\ADI42\RCPSIS.EXP

3. 	Delete the configure file SHADE.CFG.

4. 	Type 	SHADE2 	<Enter> 	to re-configure the AutoShade.

5. 	While prompting "Select display device:", choose "P386 AutoDesk Device Interface display

driver.".

6. 	While prompting "Select rendering display driver:", choose "P386 AutoDesk Device

Interface rendering driver.".

7. 	Go through all the instructions, the system would start with the desired display setting.

5  3D Studio Version 3.0 Setup

1. 	The following procedures assume that

(1) You have complete "unpack & copy" procedure.

(2) Your ADI 4.2 drivers are located in C:\ADI42.

2. 	Create your own 3D Studio V3.0 batch file (say 3DS3.BAT) and add the following

settings to it, or add the following settings to your "AUTOEXEC.BAT" file.

(1) 	SET RCPADI=C:\ADI42\RCPSIS.EXP

(2) 	SET RDPADI=C:\ADI42\RCPSIS.EXP

3. 	Execute the new 3DS batch file or reboot the computer using the new "AUTOEXEC.BAT"

as to make the new settings effective.

4. 	Change your current working directory to \3DS3

(where your 3D Studio V3.0 usually resides).

5. 	Delete original configuration file "3DADI.CFG".

6. 	Type 3DS VIBCGF	<Enter> to configure your display environment.

7. 	After the "Company Register Screen" appears, press <Enter> to continue.

8. 	The "Video Environment Configuration Screen" will appear.

Please follow the following procedures to configure your video display environment.

(1)	In "Main-Display" item,

(a)	Press <Enter> , The selection menu will appear.

(b)	In selection menu,move cursor to "RCPADI". Press <Enter>,  to select.

(2)	In "Material-Display" item,

(a)	Press	<Enter> 	The selection menu will appear.

(b)	In selection menu,	move cursor to "RCPADI". Press <Enter> to select.

(3)	In "Render-Display" item,

(a)	Press	<Enter> 	The selection menu will appear.

(b)	In selection menu,	move cursor to "RCPADI" or "RDPADI".

Press	<Enter> 	to select.

(4) 	Complete the other selections and exit configuration.

9. 	After exit configure, 3DS will boot automatically using the environment you just select.

10. 	If your previous configuration is OK, 3DS will ask you to make detail configuration for SiS

6215 drivers.If this didn't happen, please check your previous procedures or contact the

technical support people.

11. 	In the detail configuration for SiS 6215 drivers, just follow the instructions appear on the

screen and make your own choice.

If you are confused in anything, contact the technical support people for solution.

12. 	After detail configuration, you will enter the 3DS main display screen and you may begin

your 3D Studio work in the environment you make before.

13. 	Once completing the detail configuration, you may enter 3D Studio in the same configuration

simply by type \3DS3\3DS 	<Enter>  next time.

14. 	If you want to change your video configuration, just follow the procedures mentioned before

to re-configure.